Serial Graphic LCD Interface

Crownhill Associates

smart electronic solutions

###### Disclaimer

In order to comply with EMC directive 89/336/EEC, this product should not be used outside of a classroom or laboratory environment.

Any software supplied with this product is provided in an “as is” condition. No warranties, whether express, implied or statutory, including, but not limited to, implied warranties of merchantability and fitness for a particular purpose apply to this software. The company shall not, in any circumstances, be liable for special, incidental or consequential damages, for any reason whatsoever.

This product should not be used as a critical component in life support systems. Crownhill does not recommend the use of any of its products in life support applications where the failure or malfunction of the product can reasonably be expected to cause failure in the life support system or to significantly affect its safety or effectiveness.

In producing this, document and the associated hardware we have spent a great deal of time to ensure the accuracy of the information presented. We constantly strive to improve the quality of our products and documentation. Crownhill operates a quality system conforming to the requirements of BS EN ISO9001: 2000. Crownhill reserves the right to change the functionality and specifications of its products from time to time without prior notice.

If you should find any errors or omission in these documents or hardware, please contact us, we appreciate your assistance in improving our products and services.

Crownhill reserves the right to make changes to the products contained in this publication in order to improve design, performance or reliability. Crownhill assumes no responsibility for the use of any circuits described herein, conveys no license under any patent or other right, and makes no representation that the circuits are free of patent infringement.  Charts and schedules contained herein reflect representative operating parameters, and may vary depending upon a user’s specific application. While the information in this publication has been carefully checked, Crownhill shall not be liable for any damages arising as a result of any error or omission.

PICmicrotm is a trade name of Microchip Technologies Inc.

PROTONtm is a trade name of Crownhill Associates Ltd.

EPICtm is a trade name of microEngineering Labs Inc.

This document was written by Les Johnson, and published by Crownhill associates limited, Cambridge England, 2003

###### Introduction

Graphic LCDs are not only great fun to use, they also offer a very professional finish to an end product. However, cost has always been a issue, as graphic LCDs are frequently many times more expensive than their alphanumeric counterparts. But thanks to the graphic LCDs that Crownhill supplies, this is now not an issue, as they offer both affordability and flexibility.

This still leaves two chief problems when interfacing to graphic LCDs, board layout and controlling software. Board layout can be a problem because the LCD requires up to 20 pins to be wired. Controlling software can be a problem because of the complex nature of displaying anything of any use on the LCD.

The PROTON+ BASIC compiler takes the hardship out of controlling the LCD, with commands such as PRINT, PLOT, LINE, CIRCLE etc, and now the complex interface is not a problem thanks to the PROTON Graphic LCD Serial Development Board (shown below).

Using a simple RS232 serial interface, you can PRINT, SCROLL, PLOT, SAVE and LOAD screens, (plus much more) on the LCD. And not only that, but you can develop your own software then program the on-board PICmicrotm using either the PROTON+ compiler’s Bootloader, or a conventional Device programmer such as the EPICtm. These features coupled with full source code for the serial interface commands offers a truly enjoyable, educational and time saving development platform, as well as a finished product for your final design.

###### Testing the Development Board

There are two software sources supplied with the development board, one for controlling it using the PROTON+ compiler’s Serial Terminal that uses standard ASCII text; which is ideal for testing and demonstrating some of its capabilities. And one that is intended to be controlled from another microcontroller using less cumbersome binary controls. We’ll take a look at the Serial Terminal controlled software to start with, as this is already loaded on the development board when supplied.

Supply 6 to 9 Volts of power to the board via its power socket, and a short splash screen will be displayed. While the splash screen is busy, open the serial terminal built into the compiler’s IDE, by clicking on VIEW->SERIAL TERMINAL (see below).

You will be greeted with a window looking something like the screenshot below.

Now the Com port and Baud rate requires setting up. Click on the open Com icon, and a small menu will appear (see below).

Choose the appropriate Com port, according to the setup of your PC. The illustration above shows Com1 being chosen. Note: that the Com port chosen should be the same as the Com port used to download the program to the Development Board (more details later). When the Com port is chosen, another window will appear that will allow the Baud rate to be set (see below).

Set the Baud rate to 9600 (as above), and we’re ready to send some commands to the graphic LCD.

Just as a starter, type the following text on the terminal all in UPPER CASE and in quick succession.

B3F1F1001

And displayed on the graphic LCD should be a square, as shown below. OK, so it’s not an exact square, but that’s because of the aspect ratio of the graphic LCD’s pixels. They are approx 1.5 times taller than they are wide.

If a square did not appear, then try again but type quicker, and don’t forget to type in UPPER CASE.

The collection of characters told the serial LCD to draw a square “B” at X position 3F (decimal 63) and Y position 1F (decimal 31), with a RADIUS of 10 (decimal 16), and SET the pixels “01”

Note that all the values are formatted as 2 character HEX values. This is true for all the commands that have parameters, and all future discussions will be based on Hexadecimal values unless otherwise stated.

Now type ‘B3F1F1000’ and the square will disappear because the same XPOS, YPOS and RADIUS have been entered, but now a command to CLEAR the pixels was sent “00”

With that small (but crucial) test carried out, we can continue with a detailed discussion of the commands available.

###### PROTON Graphic LCD Serial Interface Commands

DISPLAY TEXT

Command: - 	T

Syntax: -	T text to display

Example: -

THELLO WORLD

The above example will display the text “HELLO WORLD”. Notice that the text did not appear until typing had finished. This is because the text is stored in a 200 element BYTE ARRAY before being sent to the LCD. However, when typing stops, a TIMEOUT occurs and the text is written to the LCD. This means that up to 200 characters can be entered in one ‘T’ command.

Notes: -

When any text reaches the end of the line, it will move down one and move the X position to 0 and carry on displaying text. Essentially performing a Carriage Return with Line Feed. When the text reaches the end of the display i.e. the end of line on the bottom line (line 07), the text will SCROLL upwards one line, and the top line will disappear.

Try typing ‘T’ followed by many characters until the bottom line is reached. The picture below shows what it could look like.

Now type in a few more characters. i.e. TUVWXYZ, and the display will scroll up one line before displaying the new text (shown below).

CLEAR the LCD

Explanation.

Clears the LCD display and positions the cursor at 0,0.

Command: - 	C

Syntax: -

C

Example: -

B3F1F1001

C

The above example will draw a square (shown earlier) , then typing the single character “C” will clear the LCD.

POSITION the CURSOR

Explanation.

Positions the cursor on the LCD normally prior to a text command being issued.

Command: - 	P

Syntax: -

P XPOS (00 to 14) LINE (00 to 07)

Example: -

C

P0003

THELLO WORLD

The above example will clear the LCD, then position the cursor at XPOS 00 and LINE 03 (remember the LCD lines count from 00 to 07) before displaying the text “HELLO WORLD”.

SET or CLEAR a SINGLE PIXEL

Explanation.

Sets or Clears a single pixel on the LCD.

Command: -	A

Syntax: -

A XPOS (00 to 7F) YPOS (00 to 3F) SET or CLEAR (00 or 01)

Example: -

C

A3F2001

The above example will clear the LCD then move to position XPOS 3F and YPOS 20 before setting the pixel near to the centre of the LCD. Altering the last two digits to 00 will clear the pixel: - A3F2000

The same pixel has now been cleared.

DRAW a CIRCLE

Explanation.

Draws or Erases a Circle on the LCD at the given X and Y coordinates.

Command: -	R

Syntax: -

R XPOS (00 to 7F) YPOS (00 to 3F) RADIUS (00 to FF) SET or CLEAR (00 or 01)

Example: -

R3F1F1001

The above example will draw a CIRCLE at XPOS 3F and YPOS 1F with a RADIUS of 10. The SET or CLEAR parameter is set to 01, so the pixels will be set. Changing the last parameter to 00 will clear the pixels. For example: - R3F1F1000 will clear the circle.

DRAW a SQUARE

Explanation.

Draws or Erases a Square on the LCD at the given X and Y coordinates.

Command: -	B

B XPOS (00 to 7F) YPOS (00 to 3F) RADIUS (00 to FF) SET or CLEAR (00 or 01)

Example: -

B3F1F1001

The above example will draw a SQUARE at XPOS 3F and YPOS 1F with a RADIUS of 10. The SET or CLEAR parameter is set to 01, so the pixels will be set. Changing the last parameter to 00 will clear the pixels. For example: - B3F1F1000 will clear the square.

DRAW a LINE

Explanation.

Draws or Erases a Line on the LCD at the given X and Y coordinates.

Command: -	L

Syntax: -

L XPOS_START (00 to 7F) YPOS_START (00 to 3F) XPOS_END (00 to 7F) YPOS_END (00 to 3F) SET or CLEAR (00 or 01)

Example: -

L00002F1F01

The above example will draw a line from XPOS_START 00 and  YPOS_START 00 to XPOS_END 2F and YPOS_END 1F.

As with all of the drawing commands, the last 2 HEX digits represent SET or CLEAR the pixel. To erase the line drawn, use: - L00002F1F00

SCROLL the DISPLAY UP

Explanation.

Scrolls the LCD up a single line. Any Text or data on the top line will disappear to make room for text or data on the bottom line. The cursor is also set to the beginning of the bottom line.

Command: -	S

Syntax: -

S

Example:-

C

P0003

THELLO WORLD

S

The above example will first clear the display (command C), then position the cursor at the beginning of line 3 (command P) and display the text “HELLO WORLD” (command T). The final character, “S”, will SCROLL the display UP one line.

SCROLL the DISPLAY DOWN

Explanation.

Scrolls the LCD down a single line. Any Text or data on the bottom line will disappear to make room for text or data on the top line.

Command: -	D

Syntax: -

D

Example:-

C

P0003

THELLO WORLD

D

The SCROLL DOWN example will first clear the display (command C), then position the cursor at the beginning of line 3 (command P) and display the text “HELLO WORLD” (command T). The final character, “D”, will SCROLL the display DOWN one line.

SMOOTH SCROLL the DISPLAY UP

Explanation.

Scrolls the LCD up by a single pixel. Any data on the top row of pixels will disappear to make room for data on the bottom row of pixels.

Command: -	N

Syntax: -

N AMOUNT of SCROLLS (00 to FF)

Example:-

C

P0005

THELLO WORLD

N3F

The SMOOTH SCROLL UP example will first clear the display (command C), then position the cursor at the beginning of line 5 (command P) and display the text “HELLO WORLD” (command T). It will then smooth scroll the text up the display 3F (63) times.

SMOOTH SCROLL the DISPLAY DOWN

Explanation.

Scrolls the LCD down by a single pixel. Any data on the bottom row of pixels will disappear to make room for data on the top row of pixels.

Command: -	O

Syntax: -

O AMOUNT of SCROLLS (00 to FF)

Example:-

C

P0002

THELLO WORLD

O3F

The SMOOTH SCROLL UP example will first clear the display (command C), then position the cursor at the beginning of line 2 (command P) and display the text “HELLO WORLD” (command T). It will then smooth scroll the text down the display 3F (63) times.

SMOOTH SCROLL the DISPLAY LEFT

Explanation.

Scrolls the LCD left by a single pixel. Any data on the right column of pixels will disappear to make room for data on the left column of pixels.

Command: -	E

Syntax: -

E AMOUNT of SCROLLS (00 to FF)

Example:-

C

THELLO WORLD <CR>

HOW ARE YOU

E3F

Note. Do not type <CR> this means press return (Carriage Return).

The SMOOTH SCROLL UP example will first clear the display (command C), then display some text (command T). It will then smooth scroll the text left across the display 3F (63) times.

SMOOTH SCROLL the DISPLAY RIGHT

Explanation.

Scrolls the LCD right by a single pixel. Any data on the left column of pixels will disappear to make room for data on the right column of pixels.

Command: -	F

Syntax: -

F AMOUNT of SCROLLS (00 to FF)

Example:-

C

THELLO WORLD <CR>

HOW ARE YOU

F3F

Note. Do not type <CR> this means press return (Carriage Return).

The SMOOTH SCROLL UP example will first clear the display (command C), then display some text (command T). It will then smooth scroll the text right 3F (63) times.

SMOOTH ROTATE the DISPLAY UP

Explanation.

Rotates the LCD upwards by a single pixel. Any data on the top row of pixels will appear on the bottom row of pixels.

Command: -	Y

Syntax: -

Y AMOUNT of ROTATES (00 to FF)

Example:-

C

P0005

THELLO WORLD

Y3F

The SMOOTH ROTATE UP example will first clear the display (command C), then position the cursor at the beginning of line 5 (command P) and display the text “HELLO WORLD” (command T). It will then smoothly rotate the text in an upward direction 3F (63) times.

SMOOTH ROTATE the DISPLAY DOWN

Explanation.

Rotates the LCD downwards by a single pixel. Any data on the bottom row of pixels will re-appear on the top row of pixels.

Command: -	V

Syntax: -

V AMOUNT of ROTATES (00 to FF)

Example:-

C

P0002

THELLO WORLD

V3F

The SMOOTH ROTATE DOWN example will first clear the display (command C), then position the cursor at the beginning of line 2 (command P) and display the text “HELLO WORLD” (command T). It will then smoothly rotate the text in a downward direction 3F (63) times.

SMOOTH ROTATE the DISPLAY LEFT

Explanation.

Rotates the LCD anticlockwise by a single pixel. Any data on the left column of pixels will re-appear on the right column of pixels.

Command: -	K

Syntax: -

K AMOUNT of ROTATES (00 to FF)

Example:-

C

THELLO WORLD <CR>

HOW ARE YOU

K7F

Note. Do not type <CR> this means press return (Carriage Return).

The SMOOTH ROTATE LEFT example will first clear the display (command C), then display some text (command T). It will then smooth rotate the display in an anticlockwise direction 80 (128) times. And the text will end where it started.

SMOOTH ROTATE the DISPLAY RIGHT

Explanation.

Rotates the LCD clockwise by a single pixel. Any data on the right column of pixels will re-appear on the left column of pixels.

Command: -	I

Syntax: -

I AMOUNT of ROTATES (00 to FF)

Example:-

C

THELLO WORLD <CR>

HOW ARE YOU

I80

Note. Do not type <CR> this means press return (Carriage Return).

The SMOOTH ROTATE RIGHT example will first clear the display (command C), then display some text (command T). It will then smooth rotate the display in a clockwise direction 80 (128) times. And the text will end where it started.

SMOOTH ROTATE PART of the DISPLAY LEFT

Explanation.

Rotates a user defined section of the LCD anticlockwise by a single pixel. Any data on the left column of pixels will re-appear on the right column of pixels.

Command: -	M

Syntax: -

M XPOS START (00 to 7F) YPOS START (00 to 3F) XPOS END (00 to 7F) YPOS END (00 to 3F) AMOUNT of ROTATES (00 to FF)

Example:-

C

THELLO WORLD <CR>

HOW ARE YOU

M05003B0237

Note. Do not type <CR> this means press return (Carriage Return).

The above example will first clear the display (command C), then display some text (command T). It will then smooth rotate part of the display in an anticlockwise direction 37 (decimal 55) times. And the text will end where it started.

SMOOTH ROTATE PART of the DISPLAY RIGHT

Explanation.

Rotates a user defined section of the LCD clockwise by a single pixel. Any data on the right column of pixels will re-appear on the left column of pixels.

Command: -	J

Syntax: -

J XPOS START (00 to 7F) YPOS START (00 to 3F) XPOS END (00 to 7F) YPOS END (00 to 3F) AMOUNT of ROTATES (00 to FF)

Example:-

C

THELLO WORLD <CR>

HOW ARE YOU

J05003B0237

Note. Do not type <CR> this means press return (Carriage Return).

The above example will first clear the display (command C), then display some text (command T). It will then smooth rotate part of the display in a clockwise direction 37 (decimal 55) times. And the text will end where it started.

SAVE a SCREEN to EEPROM MEMORY

Explanation.

Save the current LCD display to a particular portion of the I2C eeprom memory. Within a single 24LC256 serial eeprom, which contains 32 Kbytes of memory, Thirty two screens can be saved. If all 4 eeproms are fitted to the PROTON SGLCD board, then a huge 1Mbit of memory is available (128 Kbytes) which is capable of holding 128 screens. As you’ve probably gathered, a screen consists of 1 Kbytes (1024 bytes). Because of the delay required for writing to eeproms, a screen save may take a few seconds. The PROTON SGLCD board’s serial interface will acknowledge when a save is complete.

Command: -	Z

Syntax: -

Z LOCATION in MEMORY (00 to 7F)

Example: -

C

THELLO WORLD

Z10

In the example above, the LCD is first cleared (command C), and text is displayed (command T). Once the text has appeared on the LCD, the SAVE SCREEN command is issued (Z) with the location to save the screen. While the screen is being saved to eeprom memory, the PROTON SGLCD will not respond to any further commands. After a few seconds, an acknowledge character “A” will be transmitted from the PROTON SGLCD to signal the screen has been saved.

LOAD a SCREEN from EEPROM MEMORY

Explanation.

Load a screen (saved earlier) from a particular portion of the I2C eeprom memory. Within a single 24LC256 serial eeprom, which contains 32 Kbytes of memory, Thirty two screens can be stored. If all 4 eeproms are fitted to the PROTON SGLCD board, then a huge 1Mbit of memory is available (128 Kbytes) which is capable of holding 128 screens. As you’ve probably gathered, a screen consists of 1 Kbytes (1024 bytes).

Command: -	X

Syntax: -

X LOCATION in MEMORY (00 to 7F)

Example: -

C

X10

The example above should be used after the SAVE to SCREEN example has been carried out. In the example, the LCD is first cleared (command C), and the LOAD SCREEN command is issued (X) with the location to load the screen from. The previously saved screen will then be loaded on to the LCD.

Example2: -

C

X01

The PROTON SGLCD’s eeprom is preloaded with several sample screens. Example2 will load the screen stored at location 01 and the display shown below should appear.

UPLOAD a SCREEN

Explanation.

Most complex images displayed on the LCD are usually created using the PC, so a means of uploading the images to the LCD is required. That’s what the UPLOAD SCREEN command is for. It accepts a list of HEX data; where each HEX byte represents a byte on the LCD working from top left of the LCD to bottom right. Once the image is displayed on the LCD, the SAVE SCREEN command can store it in eeprom memory.

Command: -	U

Syntax: -

U

Example: -

In order to demonstrate uploading an image to the LCD, a small executable program has been created that will load an image from the PC, and upload the image to the LCD via the serial port.

Locate and run the program IMAGE_UPLOAD.EXE (found on the accompanying CDROM).

Choose the com port that the PROTON SGLD is attached to (see overleaf).

In the screen shot shown above, com port 1 was chosen. Leave the baud rate as it is for now, because it defaults to 9600, which is the default baud rate of the PROTON SGLCD. Once the com port is chosen, the UPLOAD button will be enabled.

Click on FILE, and load one of the pictures located in the same folder as the executable. The screen shot below shows the picture DOG1.BMP.

Ensure power is applied to the PROTON SGLCD, and that the splash screen has completed its journey to the bottom of the display, then press the UPLOAD button.

An image will be produced on the PROTON SGLCD’s display, scanning from top left to bottom right. Once this has completed, close the IMAGE UPLOADER, and save the screen to eeprom memory using the SAVE SCREEN command. Remember where you placed it, as we’ll use this in the next command’s example.

DOWNLOAD a SCREEN

Explanation.

Images produced on the LCD’s display can be saved to eeprom memory for long term storage, or downloaded back to the PC for inclusion into other programs. The DOWNLOAD IMAGE command, sends the data from the display in the format of CDATA statements, ready to be copied and pasted into another PROTON+ BASIC program.

Command: -	W

Syntax: -

W

Example: -

In order to demonstrate downloading an image to the PC, you must first have created or loaded one. For this example, we’ll use the previously uploaded image of the dog (see UPLOAD a SCREEN).

Open the Serial Terminal and enter the following commands: -

C

X11

W

The above commands clear the LCD (command C), and load the previously saved image from location 11. The W command then transfers the image to the Serial terminal (shown below).

LCD BACKLIGHT CONTROL

Explanation.

The graphic LCD supplied with the PROTON SGLCD has a backlight facility, but the default for this is OFF in order to conserve power. The brightness of the backlight can be controlled using the BACKLIGHT CONTROL command, and uses the microcontroller’s hardware PWM (Pulse Width Modulation) feature to further save power usage when the backlight is in use.

Command: -	G

Syntax: -

G BRIGHTNESS LEVEL (7F to FF)

Example: -

GFF

The above example will illuminate the LCD’s backlight to full brightness level.

Example 2: -

G00

Example 2 will extinguish the LCD’s backlight.

Example 3: -

GF0

Example 3 will dimly illuminate the LCD’s backlight.

Generally, a value between E0 to FF is all that’s required for the full illumination swing of the backlight.

ENABLE/DISABLE RESPONSE

Explanation.

Each command available in the PROTON SGLCD takes a finite amount of time to complete, and while this is unimportant when using the serial terminal as an interface, it is not the case if a program is automatically using the interface (such as the image uploader). What is then required is a method of knowing that the PROTON SGLCD has finished its current task and is ready for the next. This is the job of the ENABLE/DISABLE RESPONSE command. When enabled, each command will send an acknowledgement of characters <CR> “O” <CR> if all was OK, and other ACKS if something went wrong will performing the specific command.

Note. <CR> represents Carriage Return which has a value of 13.

Command: -	Q

Syntax: -

Q DISABLE or ENABLE RESPONSED (00 or 01)

Example: -

C

Q01

THELLO WORLD

The example first clears the LCD (command C), then enables RESPONSES (command Q). The PROTON SGLCD will immediately respond with the ack character “O”. Text is then displayed (command T) and the PROTON SGLCD responds with the ack character “O” when the text has finished being displayed. You should see the same results as shown in the screen shot below.

To disable the responses, then issue the command Q00. The PROTON SGLCD will give a response of “O” to signify OK.

Other responses are: -

N… Command not recognised. This will also be produced if the RETURN key is pressed, as the serial interface does not recognise this as a command.

T… Timeout occurred. If the information is not entered in a reasonable time period, a timeout will occur to allow the serial interface to look for another command.

ADJUST BAUD RATE

Explanation.

By default, the PROTON SGLCD’s serial interface works at a speed of 9600 Bits Per Second (Baud). However, it is capable of working at  much slower or higher baud rates. Up to 115200 Baud in fact.

Command: -	H

Syntax: -

H BAUD RATE VALUE (Decimal value 150 to 115200) <CR>

Example: -

C

THELLO WORLD

H19200 <CR>

< Adjust baud rate of serial terminal to 19200 before typing the following text >

THELLO AGAIN

The example above will first clear the LCD (command C), then display some text at the current baud rate (9600 by default). The baud rate will then be changed to 19200 by the H command. A carriage return must be typed after the decimal baud rate is entered. The PROTON SGLCD will then respond with an ack of “O”.

The baud rate of the PROTON SGLCD is now set to 19200, so change the baud rate of the Serial Terminal, and type in the second text message.

The new baud rate is not retained by the PROTON SGLCD after power down, therefore baud rate negotiation must be accomplished upon use.

###### Microcontroller Interface Firmware

The PROTON SGLCD will most often be accesed by another microcontroller, therefore HEX parameter values can become cumbersome to use.

By programming the PROTON SGLCD with the microcontroller firmware, named MICRO_INTERFACE.BAS, the command values stay the same, but the parameters are not required to be in ASCII HEX digits.

For example to draw a circle on the LCD, the RSOUT command could be used.

RSOUT "R" , 63 , 32 , 10 , 1

Notice how the actual command is still an ASCII “R”, but the parameters are now standard decimal values.

###### Negotiating a Response.

One thing that is profoundly different is the way the PROTON SGLCD responds to a command.

Each command takes a predetermined time to performs its task, and the controlling microcontroller needs to know when the command is finished and able to accespt another.

The PROTON SGLCD always send an acknowledge after a command, but  does so with a minimal delay of 1ms. This may be too quick for the controlling microcontroller to re-adjust itself from transmitting the command to accepting the ack from the serial port, so a method of calibraring the ack response is always a wise precaution if the controlling microcontroller is using a clock speed of less that 8MHz. Code that could be used to calibrate the ack response is listed below.

Dim RESPONSE_TIME as Byte

Dim BLANK as Byte

RESPONSE_TIME = 0		' Set initial response to 0
Repeat				' Create a loop
NEGOTIATE_RESPONSE:      ' Timeout label
Inc RESPONSE_TIME		' Increment the RESPONSE time every cycle
HRSOUT "Q",RESPONSE_TIME	' Send RESPONSE delay      
' Exit loop if response received
Until HRSIN ,{100,NEGOTIATE_RESPONSE} = "O"

The code above starts at the smallest response time which is 1, then sends the RESPONSE command of “Q”, and waits 100ms for a response of character “O”.

If a response was not received (signified by a timeout) , then the the response time is incremented until a response of character “O” is received correctly.

Once the response time its established, the PROTON SGLCD will continue using the time delay until power is removed, or another response calibration is carried out.

In tests, a response time of 2ms was found to be fairly standard using a controller operating with a 4MHz crystal.

The syntax for the RESPONSE command is: -

Q DELAY BETWEEN RESPONSES (0 to 255)

###### DOWNLOADING  a SCREEN

Another difference with the microcontroller firmware is the data received by the DOWNLOAD SCREEN command. Instead of the data being formatted for CDATA statements, it is simply transmitted as a series of values.

###### PROTON SGLCD Circuit

###### Eeprom Layout

###### ASCII Controlled Firmware Source.

' PROTON Graphic LCD RS232 serial interface
' For use with an ASCII command set
'
' Uses the low-level LCD interfacing subroutines for intensive commands
' Such as HORIZONTAL SCROLLS

DEVICE = 16F876
XTAL = 20

LCD_DTPORT = PORTB	
LCD_ENPIN = PORTA.0
LCD_RWPIN = PORTA.1
LCD_RSPIN = PORTA.2
LCD_CS2PIN = PORTA.3
LCD_CS1PIN = PORTA.4	
LCD_TYPE = GRAPHIC
INTERNAL_FONT = ON			' Use an internal font for the LCD text
        
SDA_PIN = PORTC.4				' Set serial eeprom
SCL_PIN = PORTC.3				' data/clock lines
        
HSERIAL_BAUD = 9600			' Set baud rate to 9600
HSERIAL_RCSTA = %10010000       	' Enable serial port and continuous receive
HSERIAL_TXSTA = %00100100       	' Enable transmit and asynchronous mode 
HSERIAL_CLEAR = ON			' Enable Error clearing on received characters
		
CCP1_PIN = PORTC.2
CCP2_PIN = PORTC.1
        
HBUS_BITRATE = 400			' Set I2C coms to 400 Kbits
                      
' Define the USER pinouts for the graphic LCD
Symbol GLCD_DTPORT = PORTB		' LCD's DT port
Symbol GLCD_RSPIN = PORTA.2		' LCD's RS pin
Symbol GLCD_ENPIN = PORTA.0		' LCD's EN pin
Symbol GLCD_RWPIN = PORTA.1		' LCD's RW pin
Symbol GLCD_CS1PIN = PORTA.4		' LCD's CS1 pin
Symbol GLCD_CS2PIN = PORTA.3		' LCD's CS2 pin
               
Symbol TIMEOUT = 1000			' A 1 second timeout for serial reception
Symbol RESPONSE_DELAY = 10		' Wait 10ms before sending the response
        
Dim BYTE_SAVE[8] as Byte		' Holds byte info for LCD ROTATES and SCROLLS
Dim XPOS as Byte SYSTEM			' \
Dim YPOS as Byte SYSTEM			'  \
Dim XPOS_START as Byte SYSTEM		'   | For use with low-level LCD commands
Dim YPOS_START as Byte SYSTEM		'   | Must stay as SYSTEM to be in BANK0
Dim XPOS_END as Byte SYSTEM		'  /
Dim YPOS_END as Byte SYSTEM		' /
Dim X_POSITION as Byte
Dim Y_POSITION as Byte 
Dim XPOS_LOOP as Byte 			' Used in HORIZONTAL SCROLL
Dim AMOUNT_OF_SCROLLS as Byte		' Used in HORIZONTAL SCROLL
Dim SERIAL_COMMAND as Byte		' Holds the serial command instruction
Dim DATA_LOOP as Byte			' Loop used when inputting, or outputting data
Dim DATA_BYTE as Byte
Dim RESPONSE as Byte			' Response required after each instruction
Dim RESPONSE_ENABLE as Bit		' ENABLE/DISABLE a RESPONSE after each command               
Dim SET_CLEAR as Byte			' Used by SET or CLEAR PIXEL             
Dim RADIUS as Byte			' Used by CIRCLE and SQUARE

Dim LOCATION as Byte
Dim SLAVE_ADDR as Byte
Dim ADDR as Word
Dim BAUDRATE as Word			' Save a copy of the baudrate
Dim TEMP_DWORD as Dword
Dim TEXT[201] as Byte 			' Text buffer array to hold 200 characters    
        
' Define some RESPONSES	
Symbol OK = "O"				' Standard RESPONSE        
Symbol COMMAND_NOT_RECOGNISED = "N"
Symbol TIMEOUT_ERROR = "T"

'---------------------------------------------------------------------------------                              
Delayms 200					' Wait for the PICmicro to stabilise
Goto OVER_LOW_LEVEL_SUBS		' Jump over the low level ASM subroutines
        
'---------------------------------------------------------------------------------
' LOW LEVEL assembler LCD Access Subroutines

Include "LL_INTERFACE.INC"	' Load the low level LCD routines into memory
Include "LL_SCROLLS.INC"	' Load the low level scroll routines into memory

'----[SETUP SOME VARIABLES BEFORE THE MAIN PROGRAM LOOP]--------------------------       
OVER_LOW_LEVEL_SUBS:
Clear       				' Clear all RAM before we start
ALL_DIGITAL = TRUE			' PORTA to all digital
Cls						' Clear the LCD
X_POSITION = 0				' Reset the GLOBAL XPOS of the LCD
Y_POSITION = 0				' Reset the GLOBAL YPOS of the LCD
RESPONSE_ENABLE = 0			' Default to NO responses given
Goto MAIN_LOOP				' Jump over the subroutines
      
'----[SEND AN ACKNOWLEDGE]-------------------------------------------------------- 
' If enabled, a reponse is sent to the calling devices.
' The ACKNOWLEDGE is held in variable RESPONSE
SEND_RESPONSE:
If RESPONSE_ENABLE = 1 Then		' Are RESPONSES enabled ?
 Delayms RESPONSE_DELAY			' Yes. So wait appropriate time
 Hserout [13,HEX2 RESPONSE,13]	' Send the RESPONSE value
Endif
Goto WAIT_FOR_INSTRUCTION		' Go and wait for another command
        
'----[ACT UPON A TIMEOUT ERROR]--------------------------------------------------- 
' A TIMEOUT error was encountered, while reading the instructions from the PC
TIMEOUT_ERR:
If RESPONSE_ENABLE = 1 Then		' Are RESPONSES enabled ?
 Delayms RESPONSE_DELAY			' Yes. So wait appropriate time
 Hserout [13,"T",13]			' Send a TIMEOUT error
Endif
Goto WAIT_FOR_INSTRUCTION		' Go and wait for another command
                
'----[SET/CLEAR A PIXEL]---------------------------------------------------------- 
' Command expects XPOS (00 - 7F), YPOS (00 - 3F), SET or CLEAR (00 - 01)
DO_PLOT:
' Wait for XPOS, YPOS, and SET or CLEAR commands
Hserin TIMEOUT,TIMEOUT_ERR,[HEX2 XPOS, HEX2 YPOS, HEX2 SET_CLEAR]
     
XPOS = XPOS & %01111111			' Ensure XPOS is within limits
YPOS = YPOS & %00111111			' Ensure YPOS is within limits        
If SET_CLEAR = 0 Then			' Do we require the pixel cleared ?
 UnPlot YPOS,XPOS				' Yes. So CLEAR the pixel
Else						' Otherwise SET the pixel

Plot YPOS,XPOS
Endif
Goto SEND_RESPONSE				' Send a reponse if enabled					                            
'----[SCROLL THE SCREEN UP ONE LINE]---------------------------------------------- 
SCROLL:
YPOS = 1						' Start at line 0
Repeat
 XPOS = 0						' and position 0 on the line
 Repeat    	
  DATA_BYTE = LCDREAD YPOS, XPOS  		' Read from a line             
  LCDWRITE YPOS - 1 , XPOS , [DATA_BYTE]	' And write to the line above
  Inc XPOS
 Until XPOS.7 = 1					' Until a count less than 127
 Inc YPOS
Until YPOS.3 = 1					' Until a count less than 8
Print at 7,0,REP " "\21				' Blank the bottom line
Return 
        
'----[SCROLL THE SCREEN DOWN ONE LINE]-------------------------------------------- 
SCROLL_DOWN:
YPOS = 6						' Start at line 6
Repeat
 XPOS = 0						' and position 0 on the line
 Repeat    	
  DATA_BYTE = LCDREAD YPOS, XPOS  		' Read from a line             
  LCDWRITE YPOS + 1 , XPOS , [DATA_BYTE]	' And write to the line below
  Inc XPOS
 Until XPOS.7 = 1					' Until a count less than 127
 Dec YPOS
Until YPOS = 255					' Until LINE reaches 0
Print at 0,0,REP " "\21				' Blank the top line
Return 
                       
'----[DISPLAY TEXT ON THE LCD]---------------------------------------------------- 
' Command expects text terminated by a NUL (0)
DO_TEXT: 
' Receive the DATA to display, and load the array TEXT with it.
DATA_LOOP = 0
Repeat
 Hserin 500,TEXT_TIMEOUT,[DATA_BYTE] ' Get the text serially. Display if timed out
 TEXT[DATA_LOOP] = DATA_BYTE 			' Load the array with the incoming data
 If DATA_BYTE = 0 Then SKIP_OVER		' Escape the loop if NUL found
 Inc DATA_LOOP					' Increment the loop
Until DATA_LOOP > 200			' Allow a maximum of 200 characters printable

TEXT_TIMEOUT:
TEXT[DATA_LOOP] = 0 				' Add a NUL if the loop timed out       
        
SKIP_OVER:    
DATA_LOOP = 0
Repeat
 DATA_BYTE = TEXT[DATA_LOOP]
 If DATA_BYTE = 0 Then SEND_RESPONSE	' Escape the loop if  NUL found
 If DATA_BYTE = 13 Then				' CR found ?
  X_POSITION = 0
  Inc Y_POSITION
  Goto CHECK_FOR_SCROLL				' Time to scroll ?
 Endif
 If DATA_BYTE = 10 Then				' NEW LINE found ?
  Inc Y_POSITION
  Goto CHECK_FOR_SCROLL				' Time to scroll ?

Endif
 Print at Y_POSITION , X_POSITION , DATA_BYTE
 Inc X_POSITION
 If X_POSITION > 20 Then X_POSITION = 0: Inc Y_POSITION
 CHECK_FOR_SCROLL:
 If Y_POSITION > 7 Then 			' Time to scroll ?
  Y_POSITION = 7
  Gosub SCROLL		
 Endif
 Inc DATA_LOOP
Until DATA_LOOP > 200			' Allow a maximum of 200 characters printable
Goto SEND_RESPONSE				' Send a reponse if enabled	
       
'----[POSITION THE CURSOR]-------------------------------------------------------- 
' Command expects XPOS (00 - 14), YPOS (00 - 07)
DO_POS:
Hserin TIMEOUT,TIMEOUT_ERR,[HEX2 XPOS, HEX2 YPOS]
        
If XPOS > 20 Then Goto WAIT_FOR_INSTRUCTION	' Is the XPOS range OK ?        
If YPOS > 7 Then Goto WAIT_FOR_INSTRUCTION	' Is the YPOS range OK ? 
        
X_POSITION = XPOS					' Load GLOBAL XPOS with new XPOS
Y_POSITION = YPOS					' Load GLOBAL YPOS with new YPOS
Cursor Y_POSITION , X_POSITION		' Move the cursor to position       
Goto SEND_RESPONSE				' Send a reponse if enabled

'----[DRAW A LINE]---------------------------------------------------------------- 
' Command expects XPOS-START (00 - 7F), YPOS-START (00 - 3F), XPOS-END (00 - 7F), ' YPOS-END (00 - 3F), SET or CLEAR (00 - 01)
' Any parameters out of range will be reduced in size in order to fit
DO_LINE:
Hserin TIMEOUT,TIMEOUT_ERR,[HEX2 XPOS_START, HEX2 YPOS_START,_

HEX2 XPOS_END, HEX2 YPOS_END, HEX2 SET_CLEAR]
        
XPOS_START = XPOS_START & %01111111
XPOS_END = XPOS_END & %01111111
YPOS_START = YPOS_START & %00111111
YPOS_END = YPOS_END & %00111111      
Line SET_CLEAR , XPOS_START , YPOS_START , XPOS_END , YPOS_END       
Goto SEND_RESPONSE					' Send a reponse if enabled	
        
'----[DRAW A CIRCLE]-------------------------------------------------------------- 
' Command expects XPOS (00 - 7F), YPOS (00 - 3F), RADIUS (00 - 7F) ,

' SET or CLEAR (00 - 01)
' Any parameters out of range will be reduced in size in order to fit
DO_CIRCLE:
Hserin TIMEOUT,TIMEOUT_ERR,[HEX2 XPOS, HEX2 YPOS, HEX2 RADIUS, HEX2 SET_CLEAR]
        
XPOS = XPOS & %01111111
YPOS = YPOS & %00111111       
Circle SET_CLEAR , XPOS , YPOS , RADIUS	' Draw a circle       
Goto SEND_RESPONSE				' Send a reponse if enabled

'----[DRAW A SQUARE]-------------------------------------------------------------- 
' Command expects XPOS (00 - 7F), YPOS (00 - 3F), RADIUS (00 - FF) ,

' SET or CLEAR (00 - 01)
' Any parameters out of range will be reduced in size in order to fit
DO_BOX:
Hserin TIMEOUT,TIMEOUT_ERR,[HEX2 XPOS, HEX2 YPOS, HEX2 RADIUS, HEX2 SET_CLEAR]
        
XPOS = XPOS & %01111111

YPOS = YPOS & %00111111    
Box SET_CLEAR , XPOS , YPOS , RADIUS  	' Draw a box    
Goto SEND_RESPONSE				' Send a reponse if enabled	
                
'-----[SAVE A SCREEN TO EEPROM]--------------------------------------------------- 
' Command expects the eeprom position to save screen too.
' This correcponds to a 1K (1024) byte block of I2C eeprom memory
' Starting at eeprom 1. Which is address 0
' The first 1k of this eeprom is reserved for FONT data
' With a 24LC256 eeprom, 32 screens can be saved
DO_SAVE_SCREEN:

' Receive the eeprom location data
Hserin TIMEOUT,TIMEOUT_ERR,[HEX2 LOCATION]
        
If LOCATION = 0 Then				' Make sure we do not overwrite
 Delayms 10						' Font data
 Hserout [13,"E",13]				' Stored at eeprom loacation 0
 Goto WAIT_FOR_INSTRUCTION			' Go and look for another command

Endif
If LOCATION > 128 Then Goto WAIT_FOR_INSTRUCTION ' Ignore if out of range
        
SLAVE_ADDR = %10100000				' Point to an eeprom
' Set the SLAVE address of the particular eeprom
' according to the LOCATION data received
If LOCATION > 32 Then SLAVE_ADDR = %10100010: LOCATION = LOCATION - 32
If LOCATION > 64 Then SLAVE_ADDR = %10100110: LOCATION = LOCATION - 64
If LOCATION > 96 Then SLAVE_ADDR = %10101000: LOCATION = LOCATION - 96

' A Screen of data is 1024 bytes in length
' So add the offset to the eepom address
ADDR = LOCATION * 1024
        
YPOS = 0						' Reset the outer loop counter
Repeat						' Start an outer loop for the YPOS
 XPOS = 0						' Reset the inner loop counter
 Repeat    						' Start an inner loop for the XPOS
  DATA_BYTE = LCDREAD YPOS, XPOS 		' Read the graphic LCD              
  HBusout SLAVE_ADDR, ADDR, [DATA_BYTE]	' Save the byte to eeprom
  Delayms 5						' Wait for the write to complete
  Inc ADDR						' Point to next address in eeprom memory
  Inc XPOS						' Point to the next XPOS location
 Until XPOS.7 = 1					' Loop until a count less than 127
 Inc YPOS						' Point to next YPOS (line) location
Until YPOS.3 = 1					' Loop until a count less than 8
Hserout [13,"O",13]				' Acknowledge a screen saved
        
Goto WAIT_FOR_INSTRUCTION			' Go and look for another command

'-----[LOAD A SCREEN FROM EEPROM]------------------------------------------------- 
' Command expects the eeprom position to load screen from.
' This correcponds to a 1K (1024) byte block of I2C eeprom memory
' Starting at eeprom 1. Which is address 0
' The first 1k of this eeprom is reserved for FONT data
' With a 24LC256 eeprom, 32 screens can be saved
DO_LOAD_SCREEN:

' Receive the eeprom location data
Hserin TIMEOUT,TIMEOUT_ERR,[HEX2 LOCATION]
        
If LOCATION > 128 Then Goto WAIT_FOR_INSTRUCTION	' Ignore if out of range
        
SLAVE_ADDR = %10100000						' Point to an eeprom
' Set the SLAVE address of the particular eeprom
' according to the LOCATION data received
If LOCATION > 32 Then SLAVE_ADDR = %10100010: LOCATION = LOCATION - 32
If LOCATION > 64 Then SLAVE_ADDR = %10100110: LOCATION = LOCATION - 64
If LOCATION > 96 Then SLAVE_ADDR = %10101000: LOCATION = LOCATION – 96
' A Screen of data is 1024 bytes in length
' So add the offset to the eepom address
ADDR = LOCATION * 1024
              		
YPOS = 0						' Reset the outer loop counter
Repeat						' Start an outer loop for the YPOS
 XPOS = 0						' Reset the inner loop counter
 Repeat						' Start an inner loop for the XPOS
  Hbusin SLAVE_ADDR, ADDR, [DATA_BYTE]	' Read the eeprom
  Call PBYTE					' Send byte to LCD
  'LCDWRITE YPOS, XPOS,[DATA_BYTE]		' Same code as ASM but in BASIC                                             
  Inc ADDR						' Point to next address in eeprom memory
  Inc XPOS						' Point to the next XPOS location
 Until XPOS.7 = 1					' Loop until a count less than 127
 Inc YPOS						' Point to next YPOS (line) location
Until YPOS.3 = 1					' Until a count less than 8
Goto SEND_RESPONSE				' Send a reponse if enabled	
        
'----[UPLOAD A SCREEN FROM THE SERIAL INTERFACE TO THE LCD]----------------------- 
' Displays bytes read from the serial port

DO_UPLOAD_SCREEN:		
YPOS = 0						' Reset the outer loop counter
Repeat						' Start an outer loop for the YPOS
 XPOS = 0						' Reset the inner loop counter
 Repeat						' Start an inner loop for the XPOS
  Hserin 100,WAIT_FOR_INSTRUCTION,[HEX2 DATA_BYTE] ' Get serial data
  Call PBYTE					' Send byte to LCD
  'LCDWRITE YPOS, XPOS,[DATA_BYTE]		' Same code as ASM but in BASIC

Inc XPOS						' Point to the next XPOS location
 Until XPOS.7 = 1					' Loop until a count less than 127
 Inc YPOS						' Point to next YPOS (line) location
Until YPOS.3 = 1					' Loop until a count less than 8
Goto SEND_RESPONSE 				' Send a reponse if enabled

'----[DOWNLOAD A SCREEN FROM THE LCD TO THE SERIAL INTERFACE]--------------------- 
' Sends the contents of the LCD via the serial interface
' Data is formatted as CDATA commands for easy insertion into 
' a BASIC program
DO_DOWNLOAD_SCREEN:
Hserout [13,"CDATA "]					' Transmit first CDATA command
YPOS = 0							' Reset the outer loop counter
Repeat							' Start an outer loop for the YPOS
 XPOS = 0							' Reset the inner loop counter
 Repeat							' Start an inner loop for the XPOS
  DATA_BYTE = LCDREAD YPOS, XPOS			' Read a byte from the LCD               
  Hserout [IHEX2 DATA_BYTE]				' Send it to the serial port
  ' Format data into CDATA or COMMA
  If XPOS & %00001111 <> 15 Then Hserout[","] : Else : Hserout [13,"CDATA "]
  Inc XPOS							' Point to the next XPOS location
 Until XPOS.7 = 1						' Loop until a count less than 127
 Inc YPOS						' Point to next YPOS (line) location
Until YPOS.3 = 1						' Loop until a count less than 8
Goto SEND_RESPONSE 					' Send a reponse if enabled	 
         	              
'----[INCREASE or DECREASE THE LCD BACKLIGHT BRIGHTNESS]-------------------------- 
DO_BACKLIGHT:
Hserin TIMEOUT,TIMEOUT_ERR,[HEX2 DATA_BYTE]	' Receive the brightness level
        
Hpwm 1,DATA_BYTE,15000					' PWM at 15KHz 
Goto SEND_RESPONSE					' Send a reponse if enabled
        
'----[SET THE BAUD RATE OF THE SERIAL INTERFACE]--------------------------------
DO_SET_BAUD:
' Sets the baud rate used by the USART serial interface
'SPBRG = ((((20000000) / (BAUDRATE)) + 8) / 16) - 1	;	// For BRGH = 1

' Receive the baudrate as a decimal value
Hserin TIMEOUT,TIMEOUT_ERR,[DEC BAUDRATE]	

TEMP_DWORD = 20000000 / BAUDRATE	' Calculate the value to place into SPBRG
BAUDRATE = TEMP_DWORD + 8
DATA_BYTE = BAUDRATE / 16
DATA_BYTE = DATA_BYTE - 1				' DATA_BYTE now holds new baudrate
Delayms 100							' Wait 100ms
HSerout [13,"O",13]					' Send acknowledge at old baudrate
Delayms 10							' Wait another 10ms
SPBRG = DATA_BYTE						' Set the baudrate of the USART
Goto WAIT_FOR_INSTRUCTION				' Go and look for another command 

'----[SET THE RESPONSE REPLY]----------------------------------------------------- 
' ENABLES/DISABLES a response after a command
' 00 Disables the response
' 01 Enables the response
DO_RESPONSE_SET:
Hserin TIMEOUT,TIMEOUT_ERR,[HEX2 RESPONSE_ENABLE]
Goto SEND_RESPONSE					' Send a reponse if enabled

'----[DISPLAY THE SPLASH SCREEN FOR THE LCD STARTUP]------------------------------ 
' Displays a short splash screen
' Using some of the low-level scroll subroutines

SPLASH_SCREEN:                     
Print at 0,2,"SERIAL GRAPHIC LCD"  
For XPOS_LOOP = 0 To 7
 Gosub SMOOTH_SCROLL_DOWN
 Delayms  27 - XPOS_LOOP
Next
Print at 0,6,"CROWNHILL"
For XPOS_LOOP = 0 To 7
 Gosub SMOOTH_SCROLL_DOWN
 Delayms 20 - XPOS_LOOP
Next
Print at 0,4,"WELCOME TO THE"
For XPOS_LOOP = 0 To 15
 Gosub SMOOTH_SCROLL_DOWN
 Delayms 16 - XPOS_LOOP
Next

Line 1,0,0,127,0					' Top Line
Line 1,0,63,127,63				' Bottom Line
Line 1,0,0,0,63					' Left Line
Line 1,127,0,127,63				' Right Line
Delayms 200
For XPOS_LOOP = 0 to 63
 Gosub SCROLL_LEFT
 Gosub SCROLL_LEFT
 Gosub SMOOTH_SCROLL_DOWN
Next
Cls
Cursor 0,0
Return  
          
'----[MAIN PROGRAM LOOP STARTS HERE]---------------------------------------------- 
MAIN_LOOP:             
Gosub SPLASH_SCREEN				' Display the spash screen at startup
WAIT_FOR_INSTRUCTION:
RESPONSE = OK					' Default to an OK response
XPOS_LOOP = 0				' Reset the counting loop before we enter
Hserin [SERIAL_COMMAND]				' Get the instruction
If SERIAL_COMMAND = "T" Then DO_TEXT	' Goto TEXT routine ?
If SERIAL_COMMAND = "P" Then DO_POS		' Goto POSITION CURSOR routine ? 
If SERIAL_COMMAND = "A" Then DO_PLOT	' Goto PLOT routine ?
If SERIAL_COMMAND = "L" Then DO_LINE	' Goto LINE routine ?
If SERIAL_COMMAND = "B" Then DO_BOX		' Goto BOX routine ?
If SERIAL_COMMAND = "R" Then DO_CIRCLE	' Goto CIRCLE routine ?
' Goto SCROLL UP routine ?

If SERIAL_COMMAND = "S" Then Gosub SCROLL: X_POSITION = 0: Goto SEND_RESPONSE

' Goto SCROLL DOWN routine ?

If SERIAL_COMMAND = "D" Then Gosub SCROLL_DOWN: X_POSITION = 0: Goto SEND_RESPONSE

If SERIAL_COMMAND = "E" Then 			' Do SCROLL LEFT routine ?
 Hserin TIMEOUT,TIMEOUT_ERR,[HEX2 AMOUNT_OF_SCROLLS] ' How many times ?
 Repeat						' Create a loop for the amount
  Gosub SCROLL_LEFT				' SCROLL the display LEFT
  Inc XPOS_LOOP
 Until XPOS_LOOP >= AMOUNT_OF_SCROLLS	' Have we reached the end of the loop ?
 Goto SEND_RESPONSE 			' Go wait for another command via a RESPONSE

Endif
If SERIAL_COMMAND = "F" Then 			' Do SCROLL RIGHT routine ?
 Hserin TIMEOUT,TIMEOUT_ERR,[HEX2 AMOUNT_OF_SCROLLS]
 Repeat
  Gosub SCROLL_RIGHT				' SCROLL the display RIGHT
  Inc XPOS_LOOP
 Until XPOS_LOOP >= AMOUNT_OF_SCROLLS	' Have we reached the end of the loop ?
 Goto SEND_RESPONSE 			' Go wait for another command via a RESPONSE

Endif
If SERIAL_COMMAND = "I" Then 			' Do ROTATE RIGHT routine ?
 Hserin TIMEOUT,TIMEOUT_ERR,[HEX2 AMOUNT_OF_SCROLLS]
 Repeat
  Gosub ROTATE_RIGHT				' ROTATE the display RIGHT
  Inc XPOS_LOOP
 Until XPOS_LOOP >= AMOUNT_OF_SCROLLS	' Have we reached the end of the loop ?
 Goto SEND_RESPONSE 			' Go wait for another command via a RESPONSE Endif
If SERIAL_COMMAND = "K" Then 			' Do ROTATE LEFT routine ?
 Hserin TIMEOUT,TIMEOUT_ERR,[HEX2 AMOUNT_OF_SCROLLS]
 Repeat
  Gosub ROTATE_LEFT				' ROTATE the display LEFT
  Inc XPOS_LOOP
 Until XPOS_LOOP >= AMOUNT_OF_SCROLLS	' Have we reached the end of the loop ?
 Goto SEND_RESPONSE 			' Go wait for another command via a RESPONSE Endif
If SERIAL_COMMAND = "J" Then 			' Do ROTATE PART RIGHT routine ?
 Hserin TIMEOUT,TIMEOUT_ERR,[HEX2 XPOS_START,HEX2 YPOS_START,HEX2 XPOS_END,_

HEX2 YPOS_END,HEX2 AMOUNT_OF_SCROLLS]
 Repeat
  Gosub ROTATE_PART_RIGHT			' ROTATE PART of the display RIGHT
  Inc XPOS_LOOP
 Until XPOS_LOOP >= AMOUNT_OF_SCROLLS	' Have we reached the end of the loop ?
 Goto SEND_RESPONSE 			' Go wait for another command via a RESPONSE Endif        
If SERIAL_COMMAND = "M" Then 			' Goto ROTATE PART LEFT routine ?
 Hserin TIMEOUT,TIMEOUT_ERR,[HEX2 XPOS_START,HEX2 YPOS_START,HEX2 XPOS_END,_

HEX2 YPOS_END,HEX2 AMOUNT_OF_SCROLLS]
 Repeat
  Gosub ROTATE_PART_LEFT			' ROTATE PART of the display LEFT
  Inc XPOS_LOOP
 Until XPOS_LOOP >= AMOUNT_OF_SCROLLS	' Have we reached the end of the loop ?
 Goto SEND_RESPONSE 			' Go wait for another command via a RESPONSE Endif
If SERIAL_COMMAND = "N" Then			' Do SMOOTH SCROLL UP routine ?
 Hserin TIMEOUT,TIMEOUT_ERR,[HEX2 AMOUNT_OF_SCROLLS]           
 Repeat
  Gosub SMOOTH_SCROLL_UP			' SMOOTH SCROLL the display UP
  Inc XPOS_LOOP
 Until XPOS_LOOP >= AMOUNT_OF_SCROLLS	' Have we reached the end of the loop ?
 Goto SEND_RESPONSE 			' Go wait for another command via a RESPONSE

Endif

If SERIAL_COMMAND = "O" Then			' Do SMOOTH SCROLL DOWN routine ?
 Hserin TIMEOUT,TIMEOUT_ERR,[HEX2 AMOUNT_OF_SCROLLS]
 Repeat
  Gosub SMOOTH_SCROLL_DOWN			' SMOOTH SCROLL the display DOWN
  Inc XPOS_LOOP
 Until XPOS_LOOP >= AMOUNT_OF_SCROLLS	' Have we reached the end of the loop ?
 Goto SEND_RESPONSE 			' Go wait for another command via a RESPONSE

Endif
If SERIAL_COMMAND = "V" Then			' Goto SMOOTH ROTATE DOWN routine ?
 Hserin TIMEOUT,TIMEOUT_ERR,[HEX2 AMOUNT_OF_SCROLLS]
 Repeat
  Gosub ROTATE_DOWN				' SMOOTH ROTATE the display DOWN
  Inc XPOS_LOOP
 Until XPOS_LOOP >= AMOUNT_OF_SCROLLS	' Have we reached the end of the loop ?
 Goto SEND_RESPONSE 			' Go wait for another command via a RESPONSE

Endif
If SERIAL_COMMAND = "Y" Then			' Do SMOOTH ROTATE UP routine ?
 Hserin TIMEOUT,TIMEOUT_ERR,[HEX2 AMOUNT_OF_SCROLLS]
 Repeat	
  Gosub ROTATE_UP					' SMOOTH SCROLL the display UP
  Inc XPOS_LOOP
 Until XPOS_LOOP >= AMOUNT_OF_SCROLLS	' Have we reached the end of the loop ?
 Goto SEND_RESPONSE 			' Go wait for another command via a RESPONSE
Endif
If SERIAL_COMMAND = "C" Then

Cls

X_POSITION = 0

Y_POSITION = 0

Goto SEND_RESPONSE			' Go wait for another command via a RESPONSE
Endif

If SERIAL_COMMAND = "Z" Then DO_SAVE_SCREEN	' Goto SAVE SCREEN routine ?
If SERIAL_COMMAND = "X" Then DO_LOAD_SCREEN	' Goto LOAD SCREEN routine ?
If SERIAL_COMMAND = "U" Then DO_UPLOAD_SCREEN	' Goto UPLOAD SCREEN routine ?
If SERIAL_COMMAND = "W" Then DO_DOWNLOAD_SCREEN	' Goto DOWNLOAD SCREEN routine ?
If SERIAL_COMMAND = "H" Then DO_SET_BAUD		' Goto BAUDRATE setup routine ?
If SERIAL_COMMAND = "G" Then DO_BACKLIGHT	' Goto BACKLIGHT BRIGHTNESS routine ?
If SERIAL_COMMAND = "Q" Then DO_RESPONSE_SET	' Goto RESPONSE SET routine ?
        
' Command must not be recognised, so look for another
RESPONSE = COMMAND_NOT_RECOGNISED
Goto SEND_RESPONSE

Include "FONT.INC"				' Load the LCD font table

###### Microcontroller Controlled Firmware Source

' PROTON Graphic LCD RS232 serial interface
' For use with a microcontroller
'
' Uses the low-level LCD interfacing subroutines for intensive commands
' Such as HORIZONTAL SCROLLS

DEVICE = 16F876
XTAL = 20

LCD_DTPORT = PORTB	
LCD_ENPIN = PORTA.0
LCD_RWPIN = PORTA.1
LCD_RSPIN = PORTA.2
LCD_CS2PIN = PORTA.3
LCD_CS1PIN = PORTA.4	
LCD_TYPE = GRAPHIC
INTERNAL_FONT = ON			' Use an internal font for the LCD text
        
SDA_PIN = PORTC.4				' Set serial eeprom
SCL_PIN = PORTC.3				' data/clock lines
        
HSERIAL_BAUD = 9600			' Set baud rate to 9600
HSERIAL_RCSTA = %10010000       	' Enable serial port and continuous receive
HSERIAL_TXSTA = %00100100       	' Enable transmit and asynchronous mode 
HSERIAL_CLEAR = ON			' Enable Error clearing on received characters
		
CCP1_PIN = PORTC.2
CCP2_PIN = PORTC.1
        
HBUS_BITRATE = 400			' Set I2C coms to 400 Kbits
                      
' Define the USER pinouts for the graphic LCD
Symbol GLCD_DTPORT = PORTB		' LCD's DT port
Symbol GLCD_RSPIN = PORTA.2		' LCD's RS pin
Symbol GLCD_ENPIN = PORTA.0		' LCD's EN pin
Symbol GLCD_RWPIN = PORTA.1		' LCD's RW pin
Symbol GLCD_CS1PIN = PORTA.4		' LCD's CS1 pin
Symbol GLCD_CS2PIN = PORTA.3		' LCD's CS2 pin
               
Symbol TIMEOUT = 1000			' A 1 second timeout for serial reception
Symbol RESPONSE_DELAY = 10		' Wait 10ms before sending the response
        
Dim BYTE_SAVE[8] as Byte		' Holds byte info for LCD ROTATES and SCROLLS
Dim XPOS as Byte SYSTEM			' \
Dim YPOS as Byte SYSTEM			'  \
Dim XPOS_START as Byte SYSTEM		'   | For use with low-level LCD commands
Dim YPOS_START as Byte SYSTEM		'   | Must stay as SYSTEM to be in BANK0
Dim XPOS_END as Byte SYSTEM		'  /
Dim YPOS_END as Byte SYSTEM		' /
Dim X_POSITION as Byte
Dim Y_POSITION as Byte 
Dim XPOS_LOOP as Byte 			' Used in HORIZONTAL SCROLL
Dim AMOUNT_OF_SCROLLS as Byte		' Used in HORIZONTAL SCROLL
Dim SERIAL_COMMAND as Byte		' Holds the serial command instruction
Dim DATA_LOOP as Byte			' Loop used when inputting, or outputting data
Dim DATA_BYTE as Byte
Dim RESPONSE as Byte			' Response required after each instruction
Dim RESPONSE_ENABLE as Bit		' ENABLE/DISABLE a RESPONSE after each command               
Dim SET_CLEAR as Byte			' Used by SET or CLEAR PIXEL             
Dim RADIUS as Byte			' Used by CIRCLE and SQUARE

Dim LOCATION as Byte
Dim SLAVE_ADDR as Byte
Dim ADDR as Word
Dim BAUDRATE as Word			' Save a copy of the baudrate
Dim TEMP_DWORD as Dword
Dim TEXT[201] as Byte 			' Text buffer array to hold 200 characters    
        
' Define some RESPONSES	
Symbol OK = "O"				' Standard RESPONSE        
Symbol COMMAND_NOT_RECOGNISED = "N"

'---------------------------------------------------------------------------------                              
Delayms 200					' Wait for the PICmicro to stabilise
Goto OVER_LOW_LEVEL_SUBS		' Jump over the low level ASM subroutines
        
'---------------------------------------------------------------------------------
' LOW LEVEL assembler LCD Access Subroutines

Include "LL_INTERFACE.INC"	' Load the low level LCD routines into memory
Include "LL_SCROLLS.INC"	' Load the low level scroll routines into memory

'----[SETUP SOME VARIABLES BEFORE THE MAIN PROGRAM LOOP]--------------------------       
OVER_LOW_LEVEL_SUBS:
Clear       				' Clear all RAM before we start
ALL_DIGITAL = TRUE			' PORTA to all digital
Cls						' Clear the LCD
X_POSITION = 0				' Reset the GLOBAL XPOS of the LCD
Y_POSITION = 0				' Reset the GLOBAL YPOS of the LCD
RESPONSE_ENABLE = 0			' Default to NO responses given
Goto MAIN_LOOP				' Jump over the subroutines
      
'----[SEND AN ACKNOWLEDGE]-------------------------------------------------------- 
' If enabled, a delay is implemented before reponse

' is sent to the calling devices.
' The ACKNOWLEDGE is held in variable RESPONSE
SEND_RESPONSE:
If RESPONSE_ENABLE = 1 Then		' Are RESPONSES enabled ?
 Delayms RESPONSE_DELAY			' Yes. So wait appropriate time
Endif
Hserout [RESPONSE]			' Send the RESPONSE value

Goto WAIT_FOR_INSTRUCTION		' Go and wait for another command
        
'----[ACT UPON A TIMEOUT ERROR]--------------------------------------------------- 
' A TIMEOUT error was encountered, while reading the instructions from the PC
TIMEOUT_ERR:
If RESPONSE_ENABLE = 1 Then		' Are RESPONSES enabled ?
 Delayms RESPONSE_DELAY			' Yes. So wait appropriate time 
Endif
Hserout ["T"]				' Send a TIMEOUT error

Goto WAIT_FOR_INSTRUCTION		' Go and wait for another command
                
'----[SET/CLEAR A PIXEL]---------------------------------------------------------- 
' Command expects XPOS (0 - 127), YPOS (0 - 63), SET or CLEAR (0 - 1)
DO_PLOT:
' Wait for XPOS, YPOS, and SET or CLEAR commands
Hserin TIMEOUT,TIMEOUT_ERR,[XPOS, YPOS, SET_CLEAR]
     
XPOS = XPOS & %01111111			' Ensure XPOS is within limits
YPOS = YPOS & %00111111			' Ensure YPOS is within limits        
If SET_CLEAR = 0 Then			' Do we require the pixel cleared ?
 UnPlot YPOS,XPOS				' Yes. So CLEAR the pixel
Else						' Otherwise SET the pixel

Plot YPOS,XPOS
Endif
Goto SEND_RESPONSE				' Send a reponse if enabled					                            
'----[SCROLL THE SCREEN UP ONE LINE]---------------------------------------------- 
SCROLL:
YPOS = 1						' Start at line 0
Repeat
 XPOS = 0						' and position 0 on the line
 Repeat    	
  DATA_BYTE = LCDREAD YPOS, XPOS  		' Read from a line             
  LCDWRITE YPOS - 1 , XPOS , [DATA_BYTE]	' And write to the line above
  Inc XPOS
 Until XPOS.7 = 1					' Until a count less than 127
 Inc YPOS
Until YPOS.3 = 1					' Until a count less than 8
Print at 7,0,REP " "\21				' Blank the bottom line
Return 
        
'----[SCROLL THE SCREEN DOWN ONE LINE]-------------------------------------------- 
SCROLL_DOWN:
YPOS = 6						' Start at line 6
Repeat
 XPOS = 0						' and position 0 on the line
 Repeat    	
  DATA_BYTE = LCDREAD YPOS, XPOS  		' Read from a line             
  LCDWRITE YPOS + 1 , XPOS , [DATA_BYTE]	' And write to the line below
  Inc XPOS
 Until XPOS.7 = 1					' Until a count less than 127
 Dec YPOS
Until YPOS = 255					' Until LINE reaches 0
Print at 0,0,REP " "\21				' Blank the top line
Return 
                       
'----[DISPLAY TEXT ON THE LCD]---------------------------------------------------- 
' Command expects text terminated by a NUL (0)
DO_TEXT: 
' Receive the DATA to display, and load the array TEXT with it.
DATA_LOOP = 0
Repeat
 Hserin 500,TEXT_TIMEOUT,[DATA_BYTE] ' Get the text serially. Display if timed out
 TEXT[DATA_LOOP] = DATA_BYTE 			' Load the array with the incoming data
 If DATA_BYTE = 0 Then SKIP_OVER		' Escape the loop if NUL found
 Inc DATA_LOOP					' Increment the loop
Until DATA_LOOP > 200			' Allow a maximum of 200 characters printable

TEXT_TIMEOUT:
TEXT[DATA_LOOP] = 0 				' Add a NUL if the loop timed out       
        
SKIP_OVER:    
DATA_LOOP = 0
Repeat
 DATA_BYTE = TEXT[DATA_LOOP]
 If DATA_BYTE = 0 Then SEND_RESPONSE	' Escape the loop if  NUL found
 If DATA_BYTE = 13 Then				' CR found ?
  X_POSITION = 0
  Inc Y_POSITION
  Goto CHECK_FOR_SCROLL				' Time to scroll ?
 Endif
 If DATA_BYTE = 10 Then				' NEW LINE found ?
  Inc Y_POSITION
  Goto CHECK_FOR_SCROLL				' Time to scroll ?

Endif
 Print at Y_POSITION , X_POSITION , DATA_BYTE
 Inc X_POSITION
 If X_POSITION > 20 Then X_POSITION = 0: Inc Y_POSITION
 CHECK_FOR_SCROLL:
 If Y_POSITION > 7 Then 			' Time to scroll ?
  Y_POSITION = 7
  Gosub SCROLL		
 Endif
 Inc DATA_LOOP
Until DATA_LOOP > 200			' Allow a maximum of 200 characters printable
Goto SEND_RESPONSE				' Send a reponse if enabled	
       
'----[POSITION THE CURSOR]-------------------------------------------------------- 
' Command expects XPOS (0 - 20), YPOS (0 - 7)
DO_POS:
Hserin TIMEOUT,TIMEOUT_ERR,[XPOS, YPOS]
        
If XPOS > 20 Then Goto WAIT_FOR_INSTRUCTION	' Is the XPOS range OK ?        
If YPOS > 7 Then Goto WAIT_FOR_INSTRUCTION	' Is the YPOS range OK ? 
        
X_POSITION = XPOS					' Load GLOBAL XPOS with new XPOS
Y_POSITION = YPOS					' Load GLOBAL YPOS with new YPOS
Cursor Y_POSITION , X_POSITION		' Move the cursor to position       
Goto SEND_RESPONSE				' Send a reponse if enabled

'----[DRAW A LINE]---------------------------------------------------------------- 
' Command expects XPOS-START (0 - 127), YPOS-START (0 - 63), XPOS-END (0 - 127),

' YPOS-END (0 - 127), SET or CLEAR (0 - 1)
' Any parameters out of range will be reduced in size in order to fit
DO_LINE:
Hserin TIMEOUT,TIMEOUT_ERR,[XPOS_START, YPOS_START, _END, YPOS_END, SET_CLEAR]
        
XPOS_START = XPOS_START & %01111111
XPOS_END = XPOS_END & %01111111
YPOS_START = YPOS_START & %00111111
YPOS_END = YPOS_END & %00111111      
Line SET_CLEAR , XPOS_START , YPOS_START , XPOS_END , YPOS_END       
Goto SEND_RESPONSE					' Send a reponse if enabled	
        
'----[DRAW A CIRCLE]-------------------------------------------------------------- 
' Command expects XPOS (0 - 127), YPOS (0 - 63), RADIUS (0 - 255) ,

' SET or CLEAR (0 - 1)
' Any parameters out of range will be reduced in size in order to fit
DO_CIRCLE:
Hserin TIMEOUT,TIMEOUT_ERR,[XPOS, YPOS, RADIUS, SET_CLEAR]
        
XPOS = XPOS & %01111111
YPOS = YPOS & %00111111       
Circle SET_CLEAR , XPOS , YPOS , RADIUS	' Draw a circle       
Goto SEND_RESPONSE				' Send a reponse if enabled

'----[DRAW A SQUARE]-------------------------------------------------------------- 
' Command expects XPOS (0 - 127), YPOS (0 - 63), RADIUS (0 - 255) ,

' SET or CLEAR (00 - 01)
' Any parameters out of range will be reduced in size in order to fit
DO_BOX:
Hserin TIMEOUT,TIMEOUT_ERR,[XPOS, YPOS, RADIUS, SET_CLEAR]
        
XPOS = XPOS & %01111111

YPOS = YPOS & %00111111    
Box SET_CLEAR , XPOS , YPOS , RADIUS  	' Draw a box    
Goto SEND_RESPONSE				' Send a reponse if enabled	
                
'-----[SAVE A SCREEN TO EEPROM]--------------------------------------------------- 
' Command expects the eeprom position to save screen too.
' This correcponds to a 1K (1024) byte block of I2C eeprom memory
' Starting at eeprom 1. Which is address 0
' The first 1k of this eeprom is reserved for FONT data
' With a 24LC256 eeprom, 32 screens can be saved
DO_SAVE_SCREEN:

' Receive the eeprom location data
Hserin TIMEOUT,TIMEOUT_ERR,[LOCATION]
        
If LOCATION = 0 Then				' Make sure we do not overwrite
 Delayms 10						' Font data
 Hserout [13,"E",13]				' Stored at eeprom loacation 0
 Goto WAIT_FOR_INSTRUCTION			' Go and look for another command

Endif
If LOCATION > 128 Then Goto WAIT_FOR_INSTRUCTION ' Ignore if out of range
        
SLAVE_ADDR = %10100000				' Point to an eeprom
' Set the SLAVE address of the particular eeprom
' according to the LOCATION data received
If LOCATION > 32 Then SLAVE_ADDR = %10100010: LOCATION = LOCATION - 32
If LOCATION > 64 Then SLAVE_ADDR = %10100110: LOCATION = LOCATION - 64
If LOCATION > 96 Then SLAVE_ADDR = %10101000: LOCATION = LOCATION - 96

' A Screen of data is 1024 bytes in length
' So add the offset to the eepom address
ADDR = LOCATION * 1024
        
YPOS = 0						' Reset the outer loop counter
Repeat						' Start an outer loop for the YPOS
 XPOS = 0						' Reset the inner loop counter
 Repeat    						' Start an inner loop for the XPOS
  DATA_BYTE = LCDREAD YPOS, XPOS 		' Read the graphic LCD              
  HBusout SLAVE_ADDR, ADDR, [DATA_BYTE]	' Save the byte to eeprom
  Delayms 5						' Wait for the write to complete
  Inc ADDR						' Point to next address in eeprom memory
  Inc XPOS						' Point to the next XPOS location
 Until XPOS.7 = 1					' Loop until a count less than 127
 Inc YPOS						' Point to next YPOS (line) location
Until YPOS.3 = 1					' Loop until a count less than 8
RESPONSE = "O"					' Acknowledge a screen saved     
Goto SEND_RESPONSE				' Send a reponse if enabled

'-----[LOAD A SCREEN FROM EEPROM]------------------------------------------------- 
' Command expects the eeprom position to load screen from.
' This correcponds to a 1K (1024) byte block of I2C eeprom memory
' Starting at eeprom 1. Which is address 0
' The first 1k of this eeprom is reserved for FONT data
' With a 24LC256 eeprom, 32 screens can be saved
DO_LOAD_SCREEN:

' Receive the eeprom location data
Hserin TIMEOUT,TIMEOUT_ERR,[LOCATION]
        
If LOCATION > 128 Then Goto WAIT_FOR_INSTRUCTION	' Ignore if out of range
        
SLAVE_ADDR = %10100000						' Point to an eeprom
' Set the SLAVE address of the particular eeprom
' according to the LOCATION data received
If LOCATION > 32 Then SLAVE_ADDR = %10100010: LOCATION = LOCATION - 32
If LOCATION > 64 Then SLAVE_ADDR = %10100110: LOCATION = LOCATION - 64
If LOCATION > 96 Then SLAVE_ADDR = %10101000: LOCATION = LOCATION – 96
' A Screen of data is 1024 bytes in length
' So add the offset to the eepom address
ADDR = LOCATION * 1024
              		
YPOS = 0						' Reset the outer loop counter
Repeat						' Start an outer loop for the YPOS
 XPOS = 0						' Reset the inner loop counter
 Repeat						' Start an inner loop for the XPOS
  Hbusin SLAVE_ADDR, ADDR, [DATA_BYTE]	' Read the eeprom
  Call PBYTE					' Send byte to LCD
  'LCDWRITE YPOS, XPOS,[DATA_BYTE]		' Same code as ASM but in BASIC                                             
  Inc ADDR						' Point to next address in eeprom memory
  Inc XPOS						' Point to the next XPOS location
 Until XPOS.7 = 1					' Loop until a count less than 127
 Inc YPOS						' Point to next YPOS (line) location
Until YPOS.3 = 1					' Until a count less than 8
Goto SEND_RESPONSE				' Send a reponse if enabled	
        
'----[UPLOAD A SCREEN FROM THE SERIAL INTERFACE TO THE LCD]----------------------- 
' Displays bytes read from the serial port

DO_UPLOAD_SCREEN:		
YPOS = 0						' Reset the outer loop counter
Repeat						' Start an outer loop for the YPOS
 XPOS = 0						' Reset the inner loop counter
 Repeat						' Start an inner loop for the XPOS
  Hserin 100,WAIT_FOR_INSTRUCTION,[DATA_BYTE] ' Get serial data
  Call PBYTE					' Send byte to LCD
  'LCDWRITE YPOS, XPOS,[DATA_BYTE]		' Same code as ASM but in BASIC

Inc XPOS						' Point to the next XPOS location
 Until XPOS.7 = 1					' Loop until a count less than 127
 Inc YPOS						' Point to next YPOS (line) location
Until YPOS.3 = 1					' Loop until a count less than 8
Goto SEND_RESPONSE 				' Send a reponse if enabled

'----[DOWNLOAD A SCREEN FROM THE LCD TO THE SERIAL INTERFACE]--------------------- 
' Sends the contents of the LCD via the serial interface
DO_DOWNLOAD_SCREEN: 
YPOS = 0							' Reset the outer loop counter
Repeat							' Start an outer loop for the YPOS
 XPOS = 0							' Reset the inner loop counter
 Repeat							' Start an inner loop for the XPOS
  DATA_BYTE = LCDREAD YPOS, XPOS			' Read a byte from the LCD               
  Hserout [DATA_BYTE]					' Send it to the serial port
  Inc XPOS							' Point to the next XPOS location
 Until XPOS.7 = 1						' Loop until a count less than 127
 Inc YPOS						' Point to next YPOS (line) location
Until YPOS.3 = 1						' Loop until a count less than 8
Goto SEND_RESPONSE 					' Send a reponse if enabled	 
         	              
'----[INCREASE or DECREASE THE LCD BACKLIGHT BRIGHTNESS]-------------------------- 
DO_BACKLIGHT:
Hserin TIMEOUT,TIMEOUT_ERR,[DATA_BYTE]		' Receive the brightness level
        
Hpwm 1,DATA_BYTE,15000					' PWM at 15KHz 
Goto SEND_RESPONSE					' Send a reponse if enabled
        
'----[SET THE BAUD RATE OF THE SERIAL INTERFACE]--------------------------------
DO_SET_BAUD:
' Sets the baud rate used by the USART serial interface
'SPBRG = ((((20000000) / (BAUDRATE)) + 8) / 16) - 1	;	// For BRGH = 1

' Receive the baudrate as a decimal value
Hserin TIMEOUT,TIMEOUT_ERR,[DEC BAUDRATE]	

TEMP_DWORD = 20000000 / BAUDRATE	' Calculate the value to place into SPBRG
BAUDRATE = TEMP_DWORD + 8
DATA_BYTE = BAUDRATE / 16
DATA_BYTE = DATA_BYTE - 1				' DATA_BYTE now holds new baudrate
Delayms 100							' Wait 100ms
HSerout ["O"]						' Send acknowledge at old baudrate
Delayms 10							' Wait another 10ms
SPBRG = DATA_BYTE						' Set the baudrate of the USART
Goto WAIT_FOR_INSTRUCTION				' Go and look for another command 

'----[SET THE RESPONSE REPLY]----------------------------------------------------- 
' ENABLES/DISABLES a delay before response
' 0 Disables delay
' 1 Enables delay
DO_RESPONSE_SET:
Hserin TIMEOUT,TIMEOUT_ERR,[RESPONSE_ENABLE]
Delayms 100							' Wait 100ms
RESPONSE = OK						' Send ACK
Goto WAIT_FOR_INSTRUCTION		' Go wait for another command via a RESPONSE
        
'----[SCROLL SUBROUTINE LAUNCHERS]------------------------------------------------ 
'
'----[DO SCROLL LEFT ROUTINE]----------------------------------------------------- 
DO_SCROLL_LEFT:						
XPOS_LOOP = 0						' Reset the counting loop
Hserin TIMEOUT,TIMEOUT_ERR,[AMOUNT_OF_SCROLLS] 	' How many times ?
Repeat							' Create a loop for the amount
 Gosub SCROLL_LEFT					' SCROLL the display LEFT
 Inc XPOS_LOOP
Until XPOS_LOOP >= AMOUNT_OF_SCROLLS	' Have we reached the end of the loop ?
Goto SEND_RESPONSE 			' Go wait for another command via a RESPONSE

'----[DO SCROLL RIGHT ROUTINE]---------------------------------------------------- 
DO_SCROLL_RIGHT:        
XPOS_LOOP = 0						' Reset the counting loop
Hserin TIMEOUT,TIMEOUT_ERR,[AMOUNT_OF_SCROLLS] 	' How many times ?
Repeat							' Create a loop for the amount
 Gosub SCROLL_RIGHT					' SCROLL the display RIGHT
 Inc XPOS_LOOP
Until XPOS_LOOP >= AMOUNT_OF_SCROLLS	' Have we reached the end of the loop ?
Goto SEND_RESPONSE 			' Go wait for another command via a RESPONSE	 			        
'----[DO SMOOTH SCROLL UP ROUTINE]------------------------------------------------ 
DO_SMOOTH_SCROLL_UP:
XPOS_LOOP = 0						' Reset the counting loop
Hserin TIMEOUT,TIMEOUT_ERR,[AMOUNT_OF_SCROLLS] 	' How many times ?
Repeat							' Create a loop for the amount
 Gosub SMOOTH_SCROLL_UP					' SMOOTH SCROLL the display UP
 Inc XPOS_LOOP
Until XPOS_LOOP >= AMOUNT_OF_SCROLLS	' Have we reached the end of the loop ?
Goto SEND_RESPONSE 			' Go wait for another command via a RESPONS	
'----[DO SMOOTH SCROLL DOWN ROUTINE]---------------------------------------------- 
DO_SMOOTH_SCROLL_DOWN:
XPOS_LOOP = 0						' Reset the counting loop
Hserin TIMEOUT,TIMEOUT_ERR,[AMOUNT_OF_SCROLLS] 	' How many times ?
Repeat							' Create a loop for the amount
 Gosub SMOOTH_SCROLL_DOWN				' SMOOTH SCROLL the display DOWN
 Inc XPOS_LOOP
Until XPOS_LOOP >= AMOUNT_OF_SCROLLS	' Have we reached the end of the loop ?
Goto SEND_RESPONSE 			' Go wait for another command via a RESPONSE	      
'----[DO ROTATE RIGHT ROUTINE]---------------------------------------------------- 
DO_ROTATE_RIGHT:
XPOS_LOOP = 0						' Reset the counting loop
Hserin TIMEOUT,TIMEOUT_ERR,[AMOUNT_OF_SCROLLS] 	' How many times ?
Repeat							' Create a loop for the amount
 Gosub ROTATE_RIGHT					' ROTATE the display RIGHT
 Inc XPOS_LOOP
Until XPOS_LOOP >= AMOUNT_OF_SCROLLS	' Have we reached the end of the loop ?
Goto SEND_RESPONSE 			' Go wait for another command via a RESPONSE			
'----[DO ROTATE LEFT ROUTINE]----------------------------------------------------- 
DO_ROTATE_LEFT:
XPOS_LOOP = 0						' Reset the counting loop
Hserin TIMEOUT,TIMEOUT_ERR,[AMOUNT_OF_SCROLLS] 	' How many times ?
Repeat							' Create a loop for the amount
 Gosub ROTATE_LEFT					' ROTATE the display LEFT
 Inc XPOS_LOOP
Until XPOS_LOOP >= AMOUNT_OF_SCROLLS	' Have we reached the end of the loop ?
Goto SEND_RESPONSE 			' Go wait for another command via a RESPONSE

'----[DO ROTATE UP ROUTINE]------------------------------------------------------- 
DO_ROTATE_UP:
XPOS_LOOP = 0						' Reset the counting loop
Hserin TIMEOUT,TIMEOUT_ERR,[AMOUNT_OF_SCROLLS] 	' How many times ?
Repeat							' Create a loop for the amount
 Gosub ROTATE_UP						' SMOOTH SCROLL the display UP
 Inc XPOS_LOOP
Until XPOS_LOOP >= AMOUNT_OF_SCROLLS	' Have we reached the end of the loop ?
Goto SEND_RESPONSE 			' Go wait for another command via a RESPONSE

'----[DO ROTATE DOWN ROUTINE]----------------------------------------------------- 
DO_ROTATE_DOWN:
XPOS_LOOP = 0						' Reset the counting loop
Hserin TIMEOUT,TIMEOUT_ERR,[AMOUNT_OF_SCROLLS] 	' How many times ?
Repeat							' Create a loop for the amount
 Gosub ROTATE_DOWN					' SMOOTH SCROLL the display DOWN
 Inc XPOS_LOOP
Until XPOS_LOOP >= AMOUNT_OF_SCROLLS	' Have we reached the end of the loop ?
Goto SEND_RESPONSE 			' Go wait for another command via a RESPONSE
           
'----[DO ROTATE PART RIGHT ROUTINE]----------------------------------------------- 
DO_ROTATE_PART_RIGHT:
XPOS_LOOP = 0						' Reset the counting loop
Hserin TIMEOUT,TIMEOUT_ERR,[XPOS_START,YPOS_START,_

XPOS_END, YPOS_END,AMOUNT_OF_SCROLLS]

Repeat
Gosub ROTATE_PART_RIGHT					' ROTATE PART of the display RIGHT
Inc XPOS_LOOP
Until XPOS_LOOP >= AMOUNT_OF_SCROLLS	' Have we reached the end of the loop ?
Goto SEND_RESPONSE 			' Go wait for another command via a RESPONSE	 
'----[DO ROTATE PART LEFT ROUTINE]------------------------------------------------ 
DO_ROTATE_PART_LEFT:
XPOS_LOOP = 0						' Reset the counting loop
Hserin TIMEOUT,TIMEOUT_ERR,[XPOS_START,YPOS_START,_

XPOS_END, YPOS_END,AMOUNT_OF_SCROLLS]

Repeat
Gosub ROTATE_PART_LEFT					' ROTATE PART of the display LEFT
Inc XPOS_LOOP
Until XPOS_LOOP >= AMOUNT_OF_SCROLLS	' Have we reached the end of the loop ?
Goto SEND_RESPONSE 			' Go wait for another command via a RESPONSE

'----[DISPLAY THE SPLASH SCREEN FOR THE LCD STARTUP]------------------------------ 
' Displays a short splash screen
' Using some of the low-level scroll subroutines
SPLASH_SCREEN:                     
Print at 0,2,"SERIAL GRAPHIC LCD"  
For XPOS_LOOP = 0 To 7
 Gosub SMOOTH_SCROLL_DOWN
 Delayms  27 - XPOS_LOOP
Next
Print at 0,6,"CROWNHILL"
For XPOS_LOOP = 0 To 7
 Gosub SMOOTH_SCROLL_DOWN
 Delayms 20 - XPOS_LOOP
Next
Print at 0,4,"WELCOME TO THE"
For XPOS_LOOP = 0 To 15
 Gosub SMOOTH_SCROLL_DOWN
 Delayms 16 - XPOS_LOOP
Next

Line 1,0,0,127,0					' Top Line
Line 1,0,63,127,63				' Bottom Line
Line 1,0,0,0,63					' Left Line
Line 1,127,0,127,63				' Right Line
Delayms 200
For XPOS_LOOP = 0 to 63
 Gosub SCROLL_LEFT
 Gosub SCROLL_LEFT
 Gosub SMOOTH_SCROLL_DOWN
Next

Cls
Cursor 0,0
Return  
          
'----[MAIN PROGRAM LOOP STARTS HERE]---------------------------------------------- 
MAIN_LOOP:             
Gosub SPLASH_SCREEN				' Display the spash screen at startup
WAIT_FOR_INSTRUCTION:
RESPONSE = OK					' Default to an OK response
XPOS_LOOP = 0				' Reset the counting loop before we enter
Hserin [SERIAL_COMMAND]				' Get the instruction
If SERIAL_COMMAND = "T" Then DO_TEXT	' Goto TEXT routine ?
If SERIAL_COMMAND = "P" Then DO_POS		' Goto POSITION CURSOR routine ? 
If SERIAL_COMMAND = "A" Then DO_PLOT	' Goto PLOT routine ?
If SERIAL_COMMAND = "L" Then DO_LINE	' Goto LINE routine ?
If SERIAL_COMMAND = "B" Then DO_BOX		' Goto BOX routine ?
If SERIAL_COMMAND = "R" Then DO_CIRCLE	' Goto CIRCLE routine ?
If SERIAL_COMMAND = "S" Then Gosub SCROLL : X_POSITION = 0 : Goto SEND_RESPONSE

' Goto SCROLL UP routine ?       
If SERIAL_COMMAND = "D" Then Gosub SCROLL_DOWN : X_POSITION = 0 : Goto SEND_RESPONSE

' Goto SCROLL DOWN routine ?
If SERIAL_COMMAND = "E" Then DO_SCROLL_LEFT	' Do SCROLL LEFT routine ?
If SERIAL_COMMAND = "F" Then DO_SCROLL_RIGHT	' Do SCROLL RIGHT routine ?
If SERIAL_COMMAND = "I" Then DO_ROTATE_RIGHT 	' Do ROTATE RIGHT routine ?
If SERIAL_COMMAND = "K" Then DO_ROTATE_LEFT	' Do ROTATE LEFT routine ?
If SERIAL_COMMAND = "J" Then DO_ROTATE_PART_RIGHT ' Do ROTATE PART RIGHT routine ?      
If SERIAL_COMMAND = "M" Then DO_ROTATE_PART_LEFT  ' Do ROTATE PART LEFT routine ?
If SERIAL_COMMAND = "N" Then DO_SMOOTH_SCROLL_UP  ' Do SMOOTH SCROLL UP routine ?
If SERIAL_COMMAND = "O" Then DO_SMOOTH_SCROLL_DOWN ' Do SMOOTH SCROLL DOWN routine 
If SERIAL_COMMAND = "V" Then DO_ROTATE_DOWN   ' Goto SMOOTH ROTATE DOWN routine ?
If SERIAL_COMMAND = "Y" Then DO_ROTATE_UP		' Do SMOOTH ROTATE UP routine ?
If SERIAL_COMMAND = "C" Then Cls : X_POSITION = 0 : Y_POSITION = 0 : Goto SEND_RESPONSE
If SERIAL_COMMAND = "Z" Then DO_SAVE_SCREEN	' Goto SAVE SCREEN routine ?
If SERIAL_COMMAND = "X" Then DO_LOAD_SCREEN	' Goto LOAD SCREEN routine ?
If SERIAL_COMMAND = "U" Then DO_UPLOAD_SCREEN	' Goto UPLOAD SCREEN routine ?
If SERIAL_COMMAND = "W" Then DO_DOWNLOAD_SCREEN	' Goto DOWNLOAD SCREEN routine ?
If SERIAL_COMMAND = "H" Then DO_SET_BAUD		' Goto BAUDRATE setup routine ?
If SERIAL_COMMAND = "G" Then DO_BACKLIGHT	 ' Goto BACKLIGHT BRIGHTNESS routine ?
If SERIAL_COMMAND = "Q" Then DO_RESPONSE_SET	' Goto RESPONSE SET routine ?
        
' Command must not be recognised, so look for another
RESPONSE = COMMAND_NOT_RECOGNISED
Goto SEND_RESPONSE

Include "FONT.INC"				' Load the LCD font table

###### Low-Level Scroll Subroutines Source

' LOW level SCROLL subroutines
' Written in ASM for speed and efficiency

'-----[HORIZONTAL SCROLL THE SCREEN LEFT]----------------------------------------- 
' Smooth scroll the screen LEFT  
SCROLL_LEFT:   
            
Clrf	XPOS
SL_RP1:- 	
Clrf	YPOS
SL_RP2:-	
Incf 	XPOS,W         
Call	(GBYTE)
Call	(PBYTE)  
Incf 	YPOS,F
Btfss YPOS,3
Jump 	SL_RP2
Incf 	XPOS,F
Btfss XPOS,7
Jump 	SL_RP1
           
' Clear the last row
SL_RP3:-	
Decf 	YPOS,F
Movlw 127
Movwf XPOS
Clrw
Call	(PBYTE)     	
Movfw YPOS
Btfss STATUS,Z
Jump 	SL_RP3 
Return 
'-----[HORIZONTAL SCROLL THE SCREEN RIGHT]---------------------------------------- 
' Smooth scroll the screen RIGHT     
SCROLL_RIGHT:        

' BASIC equivalent of ASM code
'XPOS = 127
'Repeat    		
'Dec XPOS
'YPOS = 0						' Start at line 0
'Repeat
'DATA_BYTE = LCDREAD YPOS , XPOS		' Read a byte
'LCDWRITE YPOS , XPOS + 1,[DATA_BYTE]	' Write the byte
'Inc YPOS
'Until YPOS.3 = 1					' Until a count less than 8       	
'Until XPOS = 0

Movlw 127
Movwf XPOS
SR_RPT1:-	
Decf 	XPOS,F
Clrf 	YPOS
SR_RPT2:-	
Movfw XPOS
Call (GBYTE)					' Read a byte
Incf 	XPOS,F
Call (PBYTE)					' Write the byte
Decf	XPOS,F

Incf 	YPOS,F
Btfss YPOS,3
Jump 	SR_RPT2
Movfw XPOS
Btfss STATUS,Z
Jump 	SR_RPT1
   
' Clear the first row  		    
SR_RP3:		
Decf 	YPOS,F
Clrw
Call 	(PBYTE)     	
Movfw YPOS
Btfss STATUS,Z
Jump 	SR_RP3
Return
 
'-----[HORIZONTAL ROTATE THE SCREEN LEFT]----------------------------------------- 
ROTATE_LEFT: 
' BASIC equivalent of ASM code
'XPOS = 0
'Repeat    		      		
'YPOS = 0						' Start at line 0
'Repeat
'DATA_LOOP = LCDREAD YPOS , 0			' Store the first byte from the screen
'DATA_BYTE = LCDREAD YPOS , XPOS + 1	' Read a byte
'LCDWRITE YPOS , XPOS ,[DATA_BYTE]		' Write the byte
'LCDWRITE YPOS , 127,[DATA_LOOP]		' Write the first byte to the last byte
'Inc YPOS
'Until YPOS.3 = 1					' Until a count less than 8

'Inc XPOS
'Until XPOS.7 = 1

' Save the bytes from the fist line on the LCD in the byte array BYTE_SAVE
Movlw	BYTE_SAVE
Movwf	FSR						' Point to beginning of array           
Clrf	YPOS
RL_RP:-		
Clrw							' Point to XPOS 0	
Call (GBYTE)
Movwf	INDF						' Store the first byte from the LCD
Incf	FSR,F						' Next bye in array            	
Incf 	YPOS,F
Btfss YPOS,3					' Loop until all 8 lines are done
Jump 	RL_RP 
            
' Rotate the bytes on the LCD                                
Clrf	XPOS
RL_RP1:- 	
Clrf	YPOS
RL_RP2:-	
Incf 	XPOS,W
Call (GBYTE)
Call (PBYTE)             
Incf YPOS,F
Btfss YPOS,3
Jump 	RL_RP2					' Loop until all 8 lines are done
Incf 	XPOS,F
Btfss XPOS,7
Jump 	RL_RP1				' Loop until all 127 bytes on a line done

' Copy the first line of bytes on the LCD to the last line via the byte array

' BYTE_SAVE'
RL_RP3:-	
Decf	FSR,F					' Move down the array by one
Decf 	YPOS,F          	
Movlw 127					' Point to XPOS 127
Movwf XPOS
Movfw	INDF					' Get a byte from the array
Call	(PBYTE) 				' Place it on the LCD               	
Movfw YPOS
Btfss STATUS,Z
Jump 	RL_RP3 				' Loop until all 8 lines are done
Return 
'-----[HORIZONTAL ROTATE THE SCREEN RIGHT]---------------------------------------- 
ROTATE_RIGHT: 
' BASIC equivalent of ASM code
'XPOS = 127
'Repeat    		
'Dec XPOS
'YPOS = 0						' Start at line 0
'Repeat
'DATA_LOOP = LCDREAD YPOS , 127		' Store the last byte from the screen
'DATA_BYTE = LCDREAD YPOS , XPOS		' Read a byte
'LCDWRITE YPOS , XPOS + 1,[DATA_BYTE]	' Write the byte
'LCDWRITE YPOS , 0,[DATA_LOOP]		' Write the byte
'Inc YPOS
'Until YPOS.3 = 1					' Until a count less than 8       	
'Until XPOS = 0
        
' Save the bytes from the LAST line on the LCD in the byte array 'BYTE_SAVE'
Movlw	BYTE_SAVE
Movwf	FSR					' Point to beginning of array           
Clrf	YPOS
RR_RP:-		
Movlw	127					' Point to XPOS 127
Call	(GBYTE)
Movwf	INDF					' Store the first byte from the LCD
Incf	FSR,F					' Next byte in array            	
Incf 	YPOS,F
Btfss 	YPOS,3			' Loop until all 8 lines are done
Jump 	RR_RP 

; Perform a SCROLL right            
Movlw 127
Movwf XPOS
RR_RPT1:-	
Decf 	XPOS,F
Clrf 	YPOS
RR_RPT2:-	
Movfw XPOS
Call 	(GBYTE)				' Read a byte
Incf 	XPOS,F
Call 	(PBYTE)				' Write the byte
Decf	XPOS,F
Incf 	YPOS,F
Btfss YPOS,3
Jump 	RR_RPT2
Movfw XPOS
Btfss STATUS,Z
Jump 	RR_RPT1

' Copy the last line of bytes on the LCD to the first line via the byte array

'BYTE_SAVE'
RR_RP3:-	
Decf	FSR,F					' Move down the array by one
Decf 	YPOS,F 						
Clrf 	XPOS					' Point to XPOS 0
Movfw	INDF					' Get a byte from the array
Call	(PBYTE) 				' Place it on the LCD               	
Movfw YPOS
Btfss STATUS,Z
Jump 	RR_RP3 				' Loop until all 8 lines are done                       
Return 
'-----[HORIZONTAL ROTATE PART OF THE SCREEN RIGHT]-------------------------------- 
ROTATE_PART_RIGHT: 		       
' BASIC equivalent of ASM code
'YPOS = 0
'Repeat

' Store the last byte from the screen	
'BYTE_SAVE[YPOS] = LCDREAD (YPOS + YPOS_START) , XPOS_END		
'Inc YPOS
'Until YPOS > YPOS_END        
'XPOS = XPOS_END
'Repeat    		
'Dec XPOS
'YPOS = YPOS_START					
'Repeat
'DATA_BYTE = LCDREAD YPOS , XPOS			' Read a byte
'LCDWRITE YPOS , XPOS + 1,[DATA_BYTE]		' Write the byte
'Inc YPOS
'Until YPOS > YPOS_END								      	
'Until XPOS = XPOS_START
        
'YPOS = YPOS_END + 1
'Repeat
'Dec YPOS
'LCDWRITE YPOS,XPOS_START,[BYTE_SAVE[YPOS]]	 Display the first byte      	
'Until YPOS = 0

YPOS_START = YPOS_START & %00000111
YPOS_END = YPOS_END & %00000111
                  	
Movlw	BYTE_SAVE
Movwf	FSR   
Clrf 	YPOS
RPR_LP:-	
Movfw YPOS_START
Addwf YPOS,F
Movfw XPOS_END
Call 	(GBYTE)
Movwf	INDF
Incf	FSR,F
Incf 	YPOS,F       
Movfw YPOS
Subwf YPOS_END,W
Btfsc STATUS,C
Jump 	RPR_LP
 
' Perform a SCROLL right            
Movfw	XPOS_END
Movwf XPOS            
RPR_RPT1:-	
Decf 	XPOS,F
Movfw	YPOS_START

Movwf	YPOS
RPR_RPT2:-	
Movfw XPOS
Call 	(GBYTE)				' Read a byte
Incf 	XPOS,F
Call 	(PBYTE)				' Write the byte
Decf	XPOS,F
Incf 	YPOS,F           
Movfw YPOS
Subwf YPOS_END,W
Btfsc STATUS,C
Jump 	RPR_RPT2
            
Movfw XPOS
Subwf XPOS_START,W
Btfss STATUS,Z
Jump RPR_RPT1
                    
Incf 	YPOS_END,W
Movwf YPOS   
Movfw XPOS_START
Movwf XPOS       
RPR_LP3:-	
Decf 	YPOS,F
Decf 	FSR,F		    
Movfw	INDF
Call 	(PBYTE)
Movfw YPOS
Btfss STATUS,Z
Jump 	RPR_LP3
Return 
'-----[HORIZONTAL ROTATE PART OF THE SCREEN LEFT]--------------------------------- 
ROTATE_PART_LEFT: 
' BASIC equivalent of ASM code		  
' XPOS_START = 0
'YPOS_START = 0        
'XPOS_END = 100
'YPOS_END = 4
'YPOS = 0
'Repeat

' Store the First byte from the screen
'BYTE_SAVE[YPOS] = LCDREAD (YPOS + YPOS_START) , XPOS_START				
'Inc YPOS
'Until YPOS > YPOS_END
'XPOS = XPOS_START
'Repeat
'YPOS = YPOS_START					
'Repeat
'DATA_BYTE = LCDREAD YPOS , XPOS + 1	' Read a byte
'LCDWRITE YPOS , XPOS,[DATA_BYTE]		' Write the byte
'Inc YPOS
'Until YPOS > YPOS_END	
'Inc XPOS							      	
'Until XPOS > XPOS_END
        
'YPOS = YPOS_END + 1
'Repeat
'Dec YPOS
'LCDWRITE YPOS,XPOS_END,[BYTE_SAVE[YPOS]]	' Display the last byte      	
'Until YPOS = 0

YPOS_START = YPOS_START & %00000111
YPOS_END = YPOS_END & %00000111
       
Movlw	BYTE_SAVE									
Movwf	FSR   				' Load FSR with address of BYTE array
Clrf 	YPOS
RPL_LP:-	
Movfw YPOS_START
Addwf YPOS,F
Movfw XPOS_START
Call 	(GBYTE)
Movwf	INDF
Incf	FSR,F
Incf 	YPOS,F       
Movfw YPOS
Subwf YPOS_END,W
Btfsc STATUS,C
Jump 	RPL_LP
                   
Movfw	XPOS_START
Movwf	XPOS
RPL_RP1:- 	
Movfw	YPOS_START
Movwf	YPOS
RPL_RP2:-	
Incf 	XPOS,W         
Call	(GBYTE)
Call	(PBYTE)  
Incf 	YPOS,F
			
Movfw YPOS
Subwf YPOS_END,W
Btfsc STATUS,C
Jump 	RPL_RP2
Incf 	XPOS,F
Movfw XPOS				‘ Until XPOS > XPOS_END
Subwf XPOS_END,W
Btfsc STATUS,C
Jump 	RPL_RP1
      
Incf 	YPOS_END,W
Movwf YPOS   
Movfw XPOS_END
Movwf XPOS       
RPL_LP3:-	
Decf 	YPOS,F
Decf 	FSR,F		    
Movfw	INDF
Call 	(PBYTE)
Movfw YPOS
Btfss STATUS,Z
Jump 	RPL_LP3
Return 
'----[SMOOTH SCROLL LCD UP]------------------------------------------------------- 
SMOOTH_SCROLL_UP:-
' BASIC equivalent of ASM code		
'YPOS = 0
'Repeat
'BYTE_SAVE[YPOS] = LCDREAD YPOS,XPOS             
'Inc YPOS          	
'Until YPOS.3 = 1
'Clrc
'Rrf BYTE_SAVE#7,F

'Rrf BYTE_SAVE#6,F
'Rrf BYTE_SAVE#5,F
'Rrf BYTE_SAVE#4,F
'Rrf BYTE_SAVE#3,F
'Rrf BYTE_SAVE#2,F
'Rrf BYTE_SAVE#1,F
'Rrf BYTE_SAVE#0,F
'YPOS = 0
'Repeat
'LCDWRITE YPOS,XPOS,[BYTE_SAVE[YPOS]]             
'Inc YPOS          	
'Until YPOS.3 = 1
      
Clrf	XPOS
SS_LP1:-
Movlw	BYTE_SAVE
Movwf	FSR
Clrf	YPOS
            
SS_LP2:-	
Movfw	XPOS
Call	(GBYTE)
Movwf	INDF
Incf	FSR,F
Incf	YPOS,F
Btfss YPOS,3
Jump 	SS_LP2
            
Clrc
Rrf 	BYTE_SAVE#7,F
Rrf 	BYTE_SAVE#6,F
Rrf 	BYTE_SAVE#5,F
Rrf 	BYTE_SAVE#4,F
Rrf 	BYTE_SAVE#3,F
Rrf 	BYTE_SAVE#2,F
Rrf 	BYTE_SAVE#1,F
Rrf 	BYTE_SAVE#0,F
            
SS_LP3:		
Decf 	YPOS,F
Decf	FSR,F
Movfw	INDF
Call 	(PBYTE)     	
Movfw YPOS
Btfss STATUS,Z
Jump 	SS_LP3
Incf 	XPOS,F
Btfss 	XPOS,7
Jump 	SS_LP1
Return 
'----[SMOOTH SCROLL LCD DOWN]----------------------------------------------------- 
SMOOTH_SCROLL_DOWN:-
' BASIC equivalent of ASM code		
'YPOS = 0
'Repeat
'BYTE_SAVE[YPOS] = LCDREAD YPOS,XPOS             
'Inc YPOS          	
'Until YPOS.3 = 1
'Clrc
'Rlf 	BYTE_SAVE#0,F
'Rlf 	BYTE_SAVE#1,F
'Rlf 	BYTE_SAVE#2,F

'Rlf 	BYTE_SAVE#3,F
'Rlf 	BYTE_SAVE#4,F
'Rlf 	BYTE_SAVE#5,F
'Rlf 	BYTE_SAVE#6,F
'Rlf 	BYTE_SAVE#7,F
'YPOS = 0
'Repeat
'LCDWRITE YPOS,XPOS,[BYTE_SAVE[YPOS]]             
'Inc YPOS          	
'Until YPOS.3 = 1
              
Clrf	XPOS
SSD_LP1:-	
Movlw	BYTE_SAVE
Movwf	FSR
Clrf	YPOS            
SSD_LP2:-	
Movfw	XPOS
Call	(GBYTE)
Movwf	INDF
Incf	FSR,F
Incf	YPOS,F
Btfss YPOS,3
Jump 	SSD_LP2
            
Clrc
Rlf 	BYTE_SAVE#0,F
Rlf 	BYTE_SAVE#1,F
Rlf 	BYTE_SAVE#2,F
Rlf 	BYTE_SAVE#3,F
Rlf 	BYTE_SAVE#4,F
Rlf 	BYTE_SAVE#5,F
Rlf 	BYTE_SAVE#6,F
Rlf 	BYTE_SAVE#7,F
            
SSD_LP3:	
Decf 	YPOS,F
Decf	FSR,F
Movfw	INDF
Call 	(PBYTE)     	
Movfw YPOS
Btfss STATUS,Z
Jump 	SSD_LP3                      
Incf 	XPOS,F
Btfss XPOS,7
Jump 	SSD_LP1
Return 
'----[SMOOTH ROTATE LCD DOWN]----------------------------------------------------- 
ROTATE_DOWN:-
		
        'YPOS = 0
        'Repeat
        'BYTE_SAVE[YPOS] = LCDREAD YPOS,XPOS             
        'Inc YPOS          	
        'Until YPOS.3 = 1
        'Rlf BYTE_SAVE#7,W
        'Rlf BYTE_SAVE#0,F
        'Rlf BYTE_SAVE#1,F
        'Rlf BYTE_SAVE#2,F
        'Rlf BYTE_SAVE#3,F
        'Rlf BYTE_SAVE#4,F
        'Rlf BYTE_SAVE#5,F
        'Rlf BYTE_SAVE#6,F
        'Rlf BYTE_SAVE#7,F
        'YPOS = 0
        'Repeat
        'LCDWRITE YPOS,XPOS,[BYTE_SAVE[YPOS]]             
        'Inc YPOS          	
        'Until YPOS.3 = 1
       
        Clrf	XPOS
SRD_LP1:-	
		Movlw	BYTE_SAVE
        Movwf	FSR
        Clrf	YPOS
            
SRD_LP2:-	
		Movfw	XPOS
        Call	(GBYTE)
        Movwf	INDF
        Incf	FSR,F
        Incf	YPOS,F
        Btfss 	YPOS,3
		Jump 	SRD_LP2
            
        Rlf		BYTE_SAVE#7,W           
        Rlf 	BYTE_SAVE#0,F
        Rlf 	BYTE_SAVE#1,F
        Rlf 	BYTE_SAVE#2,F
        Rlf 	BYTE_SAVE#3,F
        Rlf 	BYTE_SAVE#4,F
        Rlf 	BYTE_SAVE#5,F
        Rlf 	BYTE_SAVE#6,F
        Rlf 	BYTE_SAVE#7,F
            
SRD_LP3:-
		Decf 	YPOS,F
		Decf	FSR,F
        Movfw	INDF
        Call 	(PBYTE)     	
        Movfw 	YPOS
		Btfss 	STATUS,Z
		Jump 	SRD_LP3
                      
        Incf 	XPOS,F
        Btfss 	XPOS,7
		Jump 	SRD_LP1
        Return
	#if($ > 0x07FF)							; Make sure the ROTATE_DOWN routine fits inside the first PAGE af memory
    	#error "ROTATE_DOWN ROUTINE OUTSIDE THE FIRST PAGE OF MEMORY"
    #endif            
'----[SMOOTH ROTATE LCD UP]------------------------------------------------------------------------
ROTATE_UP:-
' BASIC equivalent of ASM code	
        'YPOS = 0
        'Repeat
        'BYTE_SAVE[YPOS] = LCDREAD YPOS,XPOS             
        'Inc YPOS          	
        'Until YPOS.3 = 1
        'Rrf BYTE_SAVE#0,W
        'Rrf BYTE_SAVE#7,F
        'Rrf BYTE_SAVE#6,F
        'Rrf BYTE_SAVE#5,F
        'Rrf BYTE_SAVE#4,F
        'Rrf BYTE_SAVE#3,F
        'Rrf BYTE_SAVE#2,F
        'Rrf BYTE_SAVE#1,F
        'Rrf BYTE_SAVE#0,F
        'YPOS = 0
        'Repeat
        'LCDWRITE YPOS,XPOS,[BYTE_SAVE[YPOS]]             
        'Inc YPOS          	
        'Until YPOS.3 = 1
    
        Clrf	XPOS
SRU_LP1:-	
		Movlw	BYTE_SAVE
        Movwf	FSR
        Clrf	YPOS
            
SRU_LP2:-	
		Movfw	XPOS
        Call	(GBYTE)
        Movwf	INDF
        Incf	FSR,F
        Incf	YPOS,F
        Btfss 	YPOS,3
		Jump 	SRU_LP2
            
        Rrf		BYTE_SAVE#0,W          
        Rrf 	BYTE_SAVE#7,F
        Rrf 	BYTE_SAVE#6,F
        Rrf 	BYTE_SAVE#5,F
        Rrf 	BYTE_SAVE#4,F
        Rrf 	BYTE_SAVE#3,F
        Rrf 	BYTE_SAVE#2,F
        Rrf 	BYTE_SAVE#1,F
        Rrf 	BYTE_SAVE#0,F
            
SRU_LP3:-
		Decf 	YPOS,F
		Decf	FSR,F
        Movfw	INDF
        Call 	(PBYTE)     	
        Movfw 	YPOS
		Btfss 	STATUS,Z
		Jump 	SRU_LP3
                      
        Incf 	XPOS,F
        Btfss 	XPOS,7
		Jump 	SRU_LP1
        Return 
	#if($ > 0x07FF)							; Make sure the ROTATE_UP routine fits inside the first PAGE af memory
    	#error "ROTATE_UP ROUTINE OUTSIDE THE FIRST PAGE OF MEMORY"
    #endif          
'---------------------------------------------------------------------------------