# BCProtocol Interface

Author Neil MacMullen

Date 20/4/00

Status Draft

Revision 1.0

Basic Concepts

The BCProtocol is based around the concept of bi-directional channels.  Channels must be opened before use and can only be opened once.  Data is transferred on a packet-by-packet basis and the recipient must have prepared a buffer large enough to receive the entire data packet.  This can be particularly tricky when receiving HCI data flows since the baseband layer may concatenate multiple HCI data packets and present them to the recipient as a single large packet.

Reference

void initialiseBCProtocols(BCSPStack * stack) ;

The BCProtocol layer must be initialised and passed a pointer to the stack it will use for packet transport.

bool BCopenChannel(BCChannelId chanId);

Channels must be opened once (and only once) before being used.

void BCwritePacket(BCChannelId chanId,struct _BCTransferRequest * req);

This function is used to write a packet down the specified channel.  The nature of the transfer is described by BCTransferRequest passed in

void BCreadPacket(BCChannelId chanId,struct _BCTransferRequest * req);

This function is used to read a packet from the specified channel.  The nature of the transfer is described by BCTransferRequest passed in

void BCinitTransferRequest(struct _BCTransferRequest * req,uint8 * data,uint16 len,void (*callback)(BCTransferRequest*,BCTransferStatus,void*),void * callbackParameter) ;

This function is used to initialise a transfer-request.

void BCreloadTransferRequest(struct _BCTransferRequest * req,uint8 * data,uint16 len) ;

This function is provided for convenience; it is intended that the callback and callbackParameter fields of the transfer-request will be relatively static during the life of the transfer-request and that the reload function can be used to update the data-pointer and length for the current transfer.