# Usbhidio

This is the readme file for the usbhidio application, which includes host software for communicating with USB HID-class devices.

Usbhidio is brought to you by

Jan Axelson

Lakeview Research

www.Lvr.com

(Usbhidio means Universal Serial Bus (usb) Human Interface Device (hid) Input/Output (io) application.)

## Files

In addition to this file (readme.doc), Usbhidio includes the following files:

Usbhidio.vbp is a Visual-Basic 6 project that finds the device and exchanges data with it.

Apideclarations.bas contains declarations for the Win32 API functions used in the project.

frmMain.frm is the project’s form.

usbhidio.exe is the executable file created from usbhidio.vbp.

## Rights information

The computer software provided in the usbhidio2.zip file provided as-is. You have a royalty-free right to use, modify, reproduce and distribute this software (and/or any modified version) in any way you find useful, provided that you agree that Jan Axelson and Lakeview Research have no warranty, obligation or liability for their contents!

## Project Info

'Project: usbhidio.vbp

'Version: 2.1

'Date: 7/21/05

'

'Purpose: demonstrates USB communications with a HID-class device

'Description:

'Finds an attached device that matches specific vendor and product IDs.

'Retrieves the device's capabilities.

'Sends two bytes to the device using Input reports.

'Receives two bytes from the device in Output reports.

'(For testing, the example device firmware adds 1 to the received bytes

'and sends them back.)

'A list box displays the data sent and received,

'along with error and status messages.

'Combo boxes enable you to select data to send, and to select 1-time or

'continuous transfers.

'You must set MyVendorID and MyProductID to match the values of your HID.

'For companion device firmware and PC applications for Visual Basic .NET and Visual C++,

'see my HID page at www.Lvr.com/hidpage.htm.

'Send comments, bug reports, etc. to jan@Lvr.com .

'About overlapped I/O

'

'Reading HID Input reports is done with the API function ReadFile.

'Non-overlapped ReadFile is a blocking call. If the device doesn't return the

'expected amount of data, the application hangs and waits endlessly.

'

'With overlapped I/O, the call to ReadFile returns immediately. The application uses

'WaitForSingleObject to be notified either that the data has arrived or that a timeout

'has occurred.

'WaitForSingleObject blocks the application thread but specifies a timeout value so the

'application's thread isn't blocked forever.

'

'This application has been tested on Windows 98 SE, Windows 2000, and Windows XP.